[image: image1.jpg]COUNTY OF SAN DIEGO, CALIFORNIA
BOARD OF SUPERVISORS POLICY

Subject

Policy
Number

Page






[image: image2.jpg]COUNTY OF SAN DIEGO, CALIFORNIA
BOARD OF SUPERVISORS POLICY

Subject

Policy
Number

Page




Purpose 

 
To define the County policy regarding the surfacing of the median area on a Circulation Element road when the local interest portion of the road is improved by a developer. 
 
Background 

 
San Diego County Standards require a developer to construct a 20-foot strip of pavement along each exterior edge of a Circulation Element road fronting his/her property. This is known as the local interest portion of a road. 
 
The construction of the two local interest portions of a Circulation Element road results in a 24-foot dirt strip in the middle of a collector road, a 42-foot dirt strip on a major road and a 66-foot dirt strip on a prime arterial. 
 
In the absence of any treatment of the dirt median, traffic maneuvers onto the unpaved area, creating hazardous conditions along the edge of the pavement. The elements then erode this area, damaging the road base and pavement. The result is an unsightly area and higher maintenance costs to the County. 
 
Developers and/or residents often initiate efforts to have these areas landscaped. In the case of collector roads, the landscaping eventually has to be removed, to be replaced with paving. 
 
Policy 
 
In order to define procedures for the treatment of median areas, it is the policy of the Board of Supervisors that: 

 
Whenever the construction of the local interest portion of a Circulation Element road would result in leaving an unsurfaced median between two paved traveled lanes, the developer shall, in addition to constructing the local interest portion of the road, place two inches of asphaltic surfacing on the dirt median area and place standard intermittent double yellow stripes on both sides of the asphaltic surfacing. In intersections and transitions, the developer shall construct the full pavement section to the satisfaction of the Director of Public Works and at no expense to the County. 
 
If the Director of Public Works determines all or a portion of the County interest lanes are necessary for reasons other than due to the development itself, the County may construct the 12-foot or the two 12-foot County interest lanes fronting on the local interest portion and surface the median area with two inches of asphalt concrete. 
 
Alternately, the Director of Public Works may construct a full structural section through the median. In the event that the Director of Public Works determines that only a portion of the County interest work is necessary, the developer shall treat the remaining median area in accordance with this policy. 
 
In intersections and transitions, the developer shall construct the full pavement section at no expense to the County and to the satisfaction of the Director of Public Works. 
 
In lieu of the asphalt concrete surfacing on the median area of a major or prime arterial road, the County may provide landscaping in the median area if local residents form a County Service Area to provide funding for the construction and maintenance of the landscaping. 
 
Budgeting 

 
The Director of Public Works shall include in his/her annual budget, funds sufficient to cover the estimated cost of construction of the County interest portion of Circulation Element roads resulting from the implementation of this policy. 
 
Sunset Date 
 
This policy will be reviewed for continuance by 12-31-03. 
 
Board Action 
 
5-14-74 (9) 
 
10-5-82 (58) 
 
7-26-88 (43) 
 
12-12-89 (49) 
 
7-14-99 (4)

12-4-13 (13)
 
CAO Reference 
 
1. Department of Public Works
 
Rescinded by Board of Supervisors 12/4/13 (13)











Treatment of Medians on Circulation Element Roads


											J-26	        1 of 2





Rescinded by Board of Supervisors 12/4/13 (13)











Treatment of Medians on Circulation Element Roads


											J-26	        2 of 2








