DRAFT

Groundwater Resources Investigation Report

JVR Energy Park

Jacumba Hot Springs, San Diego County, California

Lead Agency:

County of San Diego

Planning and Development Services

5510 Overland Avenue

San Diego, California 92123

Contact: Bronwyn Brown

Project Proponent:

JVR Energy Park LLC

17901 Van Karman Avenue, Suite 1050

Irvine, California 92614

Contact: Patrick Brown

Prepared by:

605 Third Street

Encinitas, California 92024

_______________________________

Trey Driscoll, PG No. 8511, CHG No. 936

November 2019

TABLE OF CONTENTS

Section	Page No.

ACRONYMs and Abbreviations	v

EXECUTIVE SUMMARY	vii

1	INTRODUCTION	1

1.1	Purpose of the Report	1

1.2	Project Location	1

1.3	Project Description	2

1.4	Project Water Demand	3

1.5	Study Area	4

1.6	Applicable Groundwater Regulations	5

2	Existing Conditions	7

2.1	Topographic and Hydrologic Setting	7

2.2	Climate	7

2.2.1	Precipitation	8

2.2.2	Evapotranspiration	11

2.3	Land Use	11

2.4	Geology and Soils	12

2.4.1	Geology	12

2.4.2	Soils	13

2.5	Hydrogeologic Units	15

2.6	Current Groundwater Demand	16

2.7	Hydrogeologic Inventory and Groundwater Level Trends	17

2.8	Water Quality	22

3	Water Quantity Impacts Analysis	25

3.1	50% Reduction of Groundwater Storage	25

3.1.1	Guidelines for Determination of Significance	25

3.1.2	Methodology	25

3.1.3	Significance of Impacts Prior to Mitigation	33

3.1.4	Mitigation Measures and Design Considerations	34

3.1.5	Conclusions	34

3.2	Well Interference and Groundwater Dependent Habitat	34

3.2.1	Guidelines for Determination of Significance	34

3.2.2	Aquifer Testing	37

3.2.3	Significance of Impacts Prior to Mitigation	42

3.2.4	Mitigation Measures and Design Considerations	44

3.2.5	Conclusions	44

4	Water Quality Impact Analysis	45

5	Summary of project Impacts and Mitigation	47

5.1	50% Reduction in Groundwater Storage	47

5.2	Well Interference	47

5.3	Groundwater-Dependent Habitat	47

5.4	Mitigation Measures	48

6	REFERENCES	49

7	List of preparers and persons anD 
organizations contacted	55

appendices

A	Well Completion Information

B	Well #2 Aquifer Test AQTESOLV Data

C	Well #3 Aquifer Test Report

EXHIBITS

2-1	Annual Precipitation Data Jacumba Rain Gauge 1963 to 2011	10

2-2	Jacumba Valley Alluvial Aquifer Groundwater Level Data July 1955 to December 2018	21

figures

1	Regional Location	57

2	Vicinity Map	59

3	Hydrologic Areas	61

4	Regional Mean Annual Precipitation	63

5	Current General Plan Land Use	65

6	Regional Geologic Map	67

7	Soils Map	69

8	Hydrogeologic Units	71

9	On-Site and Off-Site Wells	73

10	Potential Groundwater-Dependent Habitat	75

11	Well #2 24-Hour Constant Rate Test: Well #2 Drawdown	77

12	Well #2 24-Hour Constant Rate Test: Well #1 Drawdown	79

13 	Well #2 24-Hour Constant Rate Test: Well #2 Projected Drawdown	81

14	Well #2 24-Hour Constant Rate Test: Well #1 Projected Drawdown	83

15	Well #2 24-Hour Constant Rate Test: Well #2 Recovery	85

16	Well #2 24-Hour Constant Rate Test: Well #1 Recovery	87

17 	Well #3 72-Hour Constant Rate Test: Well #3 Drawdown	89

18	Well #3 72-Hour Constant Rate Test: Daley Well Hydrograph	91

19 	Well #3 72-Hour Constant Rate Test: Well #3 and Daley Well 
Projected Drawdown	93

20 	Well #3 72-Hour Constant Rate Test: Daley Well Recovery	95

21	Well #3 72-Hour Constant Rate Test: Well #3 Recovery	97

tables

1-1	Project Assessor’s Parcel Numbers	1

1-2	Estimated Construction Water Demand	3

1-3	Estimated Operational Water Demand	4

1-4	Estimated Decommission and Dismantling Water Demand	4

2-1	Precipitation Data Recorded at Jacumba Rain Gauge	8

2-2	Rain Gauges in Project Area	9

2-3	CIMIS Zone 16 Reference Evapotranspiration	11

2-4	Soil Units within the Contributing Watersheds	13

2-5	Jacumba Valley Alluvial Aquifer Existing Water Demands	17

2-6	Jacumba Valley Well Inventory	18

3-1	Jacumba Valley Alluvial Aquifer Groundwater Demand	28

3-2	Well Completion Information for Constraining Alluvial Saturated Thickness	30

3-3	Jacumba Valley Alluvial Aquifer 2018 Groundwater in Storage Estimate	32

3-4	Alluvial Aquifer Wells Within 0.5-Mile Radius of On-Site Project Wells	35

3-5	Well #2 Aquifer Test – AQTESOLV Estimated Aquifer Hydraulic Properties	39

3-6	Well #2 Distance Drawdown Calculations	40

3-7	Well #3 Distance Drawdown Calculations	42

# ACRONYMs and Abbreviations

AC		alternating current

afy		acre-feet per year

amsl		above mean sea level

bgs		below ground surface

CIMIS		California Irrigation Management Information System

County		County of San Diego

DWR		Department of Water Resources

ET		potential evapotranspiration

ETo		reference evapotranspiration

ft2/day		square feet per day

GMMP	Groundwater Monitoring and Mitigation Plan

gpm		gallons per minute

JCSD		Jacumba Community Services District

kV		kilovolt

O&M		operation and maintenance

Project		JVR Energy Park

SDG&E 	San Diego Gas & Electric

µg/L		micrograms per liter

INTENTIONALLY LEFT BLANK

# EXECUTIVE SUMMARY

Dudek has prepared this Groundwater Resources Investigation Report to evaluate the potential impact of groundwater extraction for the construction and operation of the proposed JVR Energy Park (Project) located within Jacumba Hot Springs, California.

The Project would involve the use of existing on-site wells (Well #2 and Well #3) for groundwater supply. This analysis addresses potential impacts on groundwater resources based on the Project requiring up to 112 acre-feet during construction (approximately 1 year), 10 acre-feet per year for ongoing operations and maintenance, and 50 acre-feet for decommissioning and dismantling. The significant results of this Groundwater Resource Investigation Report are as follows:

- The water demand from Well #2 and Well #3 is expected to be up to 36.5 million gallons, or 112 acre-feet, for construction to occur over an approximate 1-year period.

- The current groundwater storage in the Jacumba Valley alluvial aquifer, including the portion of the alluvial aquifer located in Mexico, is conservatively estimated to be 9,005 acre-feet based on updated groundwater level data and updated interpreted depth to bedrock using additional well logs.

- The volume of groundwater in storage would not be reduced to 50% or less than the current groundwater storage in the aquifer as a result of additional pumping for Project water supply, provided groundwater level thresholds are established to maintain groundwater in storage.

- Estimated drawdown was based on maximum groundwater production for the construction phase from either Well #2 or Well #3 at a rate of 282, 69, and 14 gallons per minute (rounded) for 90 days, 1 year, and 5 years, respectively. These adjusted production rates equal 112 acre-feet for each time period.

The nearest off-site well to Well #2 is the Highland Center Well, located 1,817 feet (0.34 miles) to the west. The estimated groundwater level drawdown at the Highland Center Well is predicted to be 0.86 feet, 0.27 feet, and 0.07 feet after 90 days, 1 year, and 5 years, respectively.

No groundwater wells are located within a 0.5-mile radius of Well #3. The nearest off-site well, Well Km, is located 3,548 feet (0.67 miles) from Well #3. The estimated groundwater level drawdown at Well Km is predicted to be 0.12 feet, 0.03 feet, and 0.01 feet after 90 days, 1 year, and 5 years, respectively.

Based on the County of San Diego well interference threshold guidance for alluvial wells, drawdown from Well #2 and Well #3 groundwater extraction would be less than significant.

The estimated drawdown at the nearest groundwater-dependent habitat from pumping Well #2 is predicted to be 0.86 feet, 0.27 feet, and 0.07 feet after 90 days, 1 year, and 5 years, respectively.

The estimated drawdown at the nearest groundwater-dependent habitat from pumping Well #3 is predicted to be 2.93 feet, 0.89 feet, and 0.22 feet after 90 days, 1 year, and 5 years, respectively.

Based on the County of San Diego groundwater-dependent habitat threshold guidance for alluvial wells, drawdown from Well #2 and Well #3 groundwater extraction would be less than significant. Estimated drawdown at the nearest groundwater-dependent habitat from pumping Well #2 and Well #3 is less than 3 feet. Furthermore, current groundwater levels in Well #3 are at least 12 feet higher than the historical low groundwater level recorded in the Jacumba Valley alluvial aquifer (Exhibit 2-2, Well K3). Therefore, drawdown as a result of Project groundwater use would be unlikely to exceed the historical low groundwater level, and impacts to groundwater-dependent habitat are anticipated to be less than significant.

- Well #2 and Well #3 are proposed to be a non-potable water source; therefore, no water quality analysis was performed for this report.

A separate Groundwater Monitoring and Mitigation Plan (GMMP) will be prepared for the proposed groundwater extraction from Well #2 and Well #3. The GMMP will establish groundwater level thresholds for off-site well interference and groundwater-dependent habitat. Additionally, the GMMP will detail requirements for ongoing groundwater level and production monitoring and reporting to the County of San Diego.

# 1	INTRODUCTION

## 1.1	Purpose of the Report

This Groundwater Resources Investigation Report was prepared on behalf of JVR Energy Park LLC by Dudek for submittal to County of San Diego (County) Planning and Development Services to satisfy groundwater resource investigation scoping requirements outlined in Guidelines for Determining Significance and Report Format and Content Requirements: Groundwater Resources (County of San Diego 2007) for the proposed JVR Energy Park (Project). This groundwater resource investigation evaluates the use of up to 112 acre-feet of water during Project construction, 10 acre-feet per year (afy) for ongoing operation and maintenance (O&M), and 50 acre-feet for decommissioning, which would occur after the Project has reached its expected lifetime (i.e., approximately 38 years). Project water would be supplied from two on-site groundwater wells.

The results of this groundwater investigation should not be relied upon for use in any other groundwater proposal subject to County review in Jacumba Hot Springs, California.

## 1.2	Project Location

The Project site is located within the Jacumba Subregional Group Area of the Mountain Empire Subregional Plan Area in unincorporated San Diego County (Figure 1, Regional Location). The Project site is located on approximately 1,345 acres in southeastern San Diego County. The Project solar facilities would be within an approximately 691-acre fenced area south of Interstate 8, east of Jacumba Hot Springs, and immediately north of the U.S./Mexico border. The Major Use Permit boundary is an approximately 655-acre area within the Project Boundary (Figure 2, Vicinity Map).

## 1.3	Project Description

The Project would have a rated capacity of up to 90 megawatts of alternating current (AC) generating capacity and would consist of photovoltaic modules fitted on single-axis solar trackers. In addition to the panels and direct current to AC conversion equipment (i.e., inverter and transformer units), the Project would include the following primary components:

- A 1,000- to 1,500-volt direct current underground collection system and a 34.5-kilovolt (kV) overhead and underground AC collection system linking the inverters to the on-site Project substation.

- An on-site collector substation.

- A 138 kV overhead and underground transmission tie-line (gen-tie line) connected on site.

A San Diego Gas & Electric (SDG&E) 138 kV switchyard adjacent to the on-site collector substation that would be used to transfer power from the on-site collector substation to the SDG&E 138 kV transmission line that traverses the Project site. The 138 kV switchyard would be designed, constructed, and operated by SDG&E.

An approximately 20-megawatt battery energy storage system located throughout the Project site.

The Project’s collector substation and the SDG&E switchyard would be sized to accommodate the full 90-megawatt (AC) solar facility and the proposed 20-megawatt energy storage system. The Project would be located entirely on private lands within unincorporated San Diego County. Upon completion, the Project would be monitored and operated off site through a supervisory control and data acquisition (SCADA) system.

Primary access to the Project site would be provided via an improved access road off of Old Highway 80. Additional access points would be provided off Carrizo Gorge Road.

## 1.4	Project Water Demand

The following discussion includes an estimate of the amount of water that would be needed for the Project during construction, ongoing O&M, and decommissioning. Groundwater demand would be supplied from on-site Well #2 and Well #3 (Figure 2). The Project would require a maximum water demand of approximately 358,436 gallons per day (approximately 250 gallons per minute [gpm]) for approximately the first 6 weeks during grading activities. The existing on-site wells have the capacity to supply the peak construction water demand. Total construction water demand is expected to be 112 acre-feet over 365 days. Estimated construction water demand by construction activity is provided in Table 1-2.

| Table 1-2 Estimated Construction Water Demand |  |  |
| --- | --- | --- |
| Activity | Total Estimated Water Demand | Total Estimated Water Demand (acre-feet)1 |
| Site preparation (clearing, grubbing, grinding, and dust control) | 1 acre-foot per day for 28 days | 28 |
| Grading | 1.1 acre-foot per day for 60 days | 64 |
| Dust abatement2 | Approximately 47,000 gallons per day for 125 days | 18 |
| Other construction needs | Water necessary for other construction needs, such as filling tanks for fire protection; washing stations for vehicles/equipment (noxious weed mitigation); the 1,500-foot gen-tie line; and concrete hydration requirements for substation, inverter, and other facility foundations (e.g., fencing, lighting). | 2 |
| Total Construction Water Use |  | 112 |

1	1 acre-foot equals 325,851 gallons.

2 	Dust abatement is included in the estimate for initial site preparation (first 40 days); therefore, general dust abatement was assumed to occur over 104 days (i.e., the remainder of the construction phase).

During operation, the Project would require water for panel washing approximately twice a year. Operational water demand is estimated to be 10 afy (Table 1-3).

| Table 1-3 Estimated Operational Water Demand |  |
| --- | --- |
| Activity | Estimated Water Demand (acre-feet) |
| Panel washing and other maintenance activities | 10 |
| Total Water Use per Year | 10 |

It is estimated that the amount of water necessary to decommission the Project would be less than that required for construction, because there would be no need to use water for concrete mixing or to hydrate and compact on-site fills. The activities associated with decommissioning would not include grading, and based on the estimates calculated for construction, water demand for decommissioning dust abatement would be approximately 40 acre-feet of water total. Additional equipment washing and modest compaction needs, if necessary, would require approximately 10 acre-feet. The total estimated water demand for decommissioning is approximately 50 acre-feet (Table 1-4).

| Table 1-4 Estimated Decommission and Dismantling Water Demand |  |
| --- | --- |
| Activity | Total Estimated Water Demand (acre-feet) |
| Decommission Dust Abatement | 40 |
| Equipment Washing and Compaction | 10 |
| Total Water Use | 50 |

## 1.5	Study Area

The study area for the purpose of discussions of groundwater storage is the Quaternary alluvium, referred to as the Jacumba Valley alluvial aquifer. The study area for the purpose of discussions of recharge consists of Flat Creek (which includes Blue Angel Peak and an unnamed subwatershed; naming convention adopted from Swenson 1981), Boundary Creek, and a portion of Walker Canyon-Carrizo Creek subwatersheds (referred to in this report as “contributing watersheds”). The study area for the purpose of well interference is the 0.5-mile radius around Well #2 and around Well #3.

## 1.6	Applicable Groundwater Regulations

The County of San Diego Guidelines for Determining Significance and Report Format and Content Requirements: Groundwater Resources (County Guidelines) contain a series of significance thresholds for groundwater quantity and groundwater quality (County of San Diego 2007). The County Guidelines contain the following guidelines that, if met, would be considered a significant impact to local groundwater resources as a result of Project implementation.

To evaluate impacts to groundwater resources, a water balance analysis is typically required; the following guideline for determining significance is typically used (County of San Diego 2007):

For proposed projects in fractured rock and sedimentary basins, groundwater impacts will be considered significant if a soil moisture balance, or equivalent analysis, conducted using a minimum of 30 years of precipitation data, including drought periods, concludes that at any time groundwater in storage is reduced to a level of 50% or less as a result of groundwater extraction.

To evaluate off-site well interference in alluvial wells, the following guideline for determining significance is typically used (County of San Diego 2007):

As an initial screening tool, off-site well interference will be considered a significant impact if after a five year projection of drawdown, the results indicate a decrease in water level of 5 feet or more in the off-site wells. If site-specific data indicates alluvium or sedimentary rocks exist which substantiate a saturated thickness greater than 100 feet in off-site wells, a decrease in saturated thickness of 5% or more in the off-site wells would be considered a significant impact.

To evaluate groundwater quality impacts, the following guideline for determining significance is typically used (County of San Diego 2007):

Groundwater resources for proposed projects requiring a potable water source must not exceed the Primary State or Federal Maximum Contaminant Levels (MCLs) for applicable contaminants. Proposed projects that cannot demonstrate compliance with applicable MCLs will be considered to have a significant impact. In general, projects will be required to sample water supply wells for nitrate, bacteria (fecal and total coliform), and radioactive elements. Projects may be required to sample other contaminants of potential concern depending on the geographical location within the County.

The Project does not propose to use groundwater as a potable water source, so the above guideline for determining significance does not apply.

To evaluate impacts to groundwater-dependent habitat, the following guideline for determining significance is typically used (County of San Diego 2010a):

The project would draw down the groundwater table to the detriment of groundwater-dependent habitat, typically a drop of 3 feet or more from historical low groundwater levels.

# 2	Existing Conditions

## 2.1	Topographic and Hydrologic Setting

Jacumba Hot Springs is located in the southeastern corner of San Diego County and is bordered by Imperial County to the east and Mexico to the south (Figures 1 and 2). The general topography of the Project site within the Jacumba Valley is gently rolling. The Project site has been previously disturbed for agricultural purposes. The elevation range within the Project site is from approximately 2,715 feet to 2,915 feet above mean sea level (amsl).

The contributing watersheds to the Project site cover 70,868 acres (111 square miles), with 76% located in Baja California, Mexico. The contributing watersheds are located in the Upper Carrizo Creek watershed as defined by the U.S. Geological Survey (Figure 3, Hydrologic Areas). The majority of flow from Mexico north into the Jacumba Valley is derived from the Flat Creek subwatershed, which includes Blue Angel Peak and an unnamed subwatershed. The subwatersheds predominantly located in the United States are the Boundary Creek and Walker Canyon-Carrizo Creek subwatersheds. The Jacumba Valley ultimately drains through a narrow constriction north of Jacumba Hot Springs known as the Carrizo Gorge.

The Flat Creek subwatershed consists of approximately 51,052 acres, with 134 acres (0.26%) of the watershed located in the United States. The Flat Creek subwatershed ranges from 4,774 feet amsl at its headwaters along the Sierra Juarez Mountains to 2,800 feet amsl near the international border. The Boundary Creek subwatershed consists of approximately 12,535 acres, with 10,106 acres (81%) of the watershed located in the United States. The Boundary Creek subwatershed ranges from 4,240 feet amsl and its headwaters along the Tecate Divide to 2,788 feet amsl. The Walker Canyon-Carrizo Creek subwatershed consist of approximately 7,281 acres, with 6,927 acres (95%) of the watershed located in the United States. The Walker Canyon-Carrizo Creek subwatershed ranges from 4,097 feet amsl at Table Mountain to 2,713 feet amsl at the north end of the Project site (Google Earth 2015).

## 2.2	Climate

Jacumba Hot Springs experiences warm summer months and cool winters. Average temperatures vary greatly within the region. Mean maximum temperatures in the summer months reach the high-80s to low-90s degrees Fahrenheit. Temperatures may fall below freezing in the winter, with snow levels occasionally below 2,500 feet (WRCC 2019).

### 2.2.1	Precipitation

The precipitation that recharges the Project site falls within the contributing watersheds. Monthly precipitation records were obtained from the County for a rain gauge previously located in Jacumba at 32°37' North latitude, 116°11' West longitude, and an elevation of 2,800 feet. The period of record available is from March 1963 until March 2011. Table 2-1 provides average monthly precipitation data, as well as the highest and lowest monthly precipitation for the Jacumba rain gauge (Allan 2013).

| Table 2-1 Precipitation Data Recorded at Jacumba Rain Gauge |  |  |  |
| --- | --- | --- | --- |
| Month | Rainfall (inches) for 1963–2011a |  |  |
|  | Average | Highest / Year | Lowestb |
| January | 1.45 | 5.79 / 1983 | 0 |
| February | 1.66 | 10.86 / 1993 | 0 |
| March | 1.82 | 6.76 / 1998 | 0 |
| April | 1.45 | 7.13 / 1991 | 0 |
| May | 0.50 | 2.38 / 1965 | 0 |
| June | 0.19 | 2.24 / 1981 | 0 |
| July | 0.06 | 0.96 / 1984 | 0 |
| August | 0.45 | 3.97 / 1984 | 0 |
| September | 0.50 | 3.48 / 1992 | 0 |
| October | 0.37 | 4.58 / 1976 | 0 |
| November | 0.60 | 4.37 / 2004 | 0 |
| December | 0.85 | 3.82 / 1965 | 0 |
| Year | 9.64 | 22.16 / 1982–1983 | 2.26 |

Source: Allan 2013.

Notes: Jacumba rain gauge was located at N 32°37', W 116°11', at an elevation of 2,800 feet.

- Jacumba rain gauge was active from 1963 to 2011.

- Lowest monthly recorded precipitation data is not available due to data gaps.

For the period from 1963 through 2011, the average annual precipitation at the Jacumba rain gauge was approximately 9.64 inches, with 85% of the precipitation occurring between October and April. Annual precipitation totals at the Jacumba rain gauge varied from a high of 22.16 inches in the 1982–1983 water year to a low of 2.26 inches in the 2001–2002 water year (Allan 2013) (see Exhibit 2-1).

Precipitation records from four nearby rain gauges were obtained to determine annual average rainfall within the watersheds. The rain gauges are located in Boulevard (two stations), Tierra del Sol, and Jacumba. The locations, elevations, years of operation, mean annual rainfall, and source of data are provided in Table 2-2.

| Table 2-2 Rain Gauges in Project Area |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Station | Location | Elevation (feet amsl) | Years of Operation | Average Annual Rainfall (inches) | Source |
| Boulevard 1 | N 32°40', W 116°17' | 3,353 | 1924 to 1967 | 14.8 | NOAA1 |
| Boulevard 2 | N 32°40', W 116°18' | 3,600 | 1969 to 1994 | 17.0 | NOAA |
| Tierra del Sol | N 32°39', W 116°19' | 4,000 | 1971 to 2017 | 10.8 | County2 |
| Jacumba | N 32°37', W 116°11' | 2,800 | 1963 to 2011 | 9.64 | County3 |

1 	NOAA 2011

2 	Allan 2014

3 	Allan 2013

amsl = above mean sea level

The isohyetal map of annual precipitation, developed by Swenson (1981), shows that the majority of the Flat Creek subwatershed receives an average of 11 inches of precipitation per year (Figure 4, Regional Mean Annual Precipitation). The lower elevations of the subwatershed receive an average of 9 inches of precipitation per year. Mean annual precipitation, as determined from the County of San Diego map entitled “Groundwater Limitations Map” on file with the Clerk of the Board of Supervisors as Document No. 195172, indicates the Walker Canyon-Carrizo Creek subwatershed receives an average of 9 inches of precipitation per year. The Groundwater Limitations Map indicates that the majority of the Boundary Creek subwatershed receives an average of 14 inches of precipitation per year at its highest elevation, and an average of 9 inches of precipitation per year at its lowest (County of San Diego 2004). The County Groundwater Limitations Map roughly concurs with those developed by Swenson (1981) (Figure 4).

The average annual precipitation of 9 inches at the Project site also roughly agrees with the average precipitation calculated for the Jacumba rain gauge between 1963 and 2011 of 9.64 inches (Allan 2013). The Jacumba rain gauge was located at the lowest elevation in the Flat Creek subwatershed.

Exhibit 2-1
Annual Precipitation Data Jacumba Rain Gauge 1963 to 2011

Source: Allan 2013.

Note: Station located at N 32°37', W 116°11' at an elevation of 2,800 feet

### 2.2.2	Evapotranspiration

According to the State of California Reference Evapotranspiration Map developed by the California Irrigation Management Information System (CIMIS), the Project site is located in Evapotranspiration Zone 16, with an average of 62.5 inches of reference evapotranspiration (ETo) per year (CIMIS 1999). Table 2-3 presents ETo by month in CIMIS Zone 16. The annual 62.5 inches of ETo is based on potential evapotranspiration (ET) from turf grass/alfalfa crop, which assumes a continuous source of moisture and does not consider summer plant dormancy. Therefore, ETo is an overestimation of actual ET, which varies with the vegetation type. To account for variations in plant water consumption and more accurately assess ET, a crop coefficient can be applied to ETo. Plants that consume less water have lower crop coefficients. Drought-tolerant plants and native vegetation have a crop coefficient of approximately 0.3 (DWR and UCCE 2000). Using this crop coefficient, the annual estimated ET for the Project site is 62.5 inches x 0.3 = 18.75 inches.

| Table 2-3 CIMIS Zone 16 Reference Evapotranspiration |  |
| --- | --- |
| Month | Reference Evapotranspiration (inches) |
| January | 1.55 |
| February | 2.52 |
| March | 4.03 |
| April | 5.7 |
| May | 7.75 |
| June | 8.7 |
| July | 9.3 |
| August | 8.37 |
| September | 6.3 |
| October | 4.34 |
| November | 2.4 |
| December | 1.55 |
| Year | 62.51 |

Source: CIMIS 1999

## 2.3	Land Use

According to the San Diego County General Plan, Jacumba Hot Springs is located within the Mountain Empire Subregional Plan area (County of San Diego 2011). Land use designations within a 0.5-mile radius of Well #2 consist of single-family residential, spaced rural residential, airstrip, communications and utilities, railroad right-of-way, road right-of-way, neighborhood shopping center, religious facility, library, other public services, and open space park or preserve (see Figure 5, Current General Plan Land Use). Land use designations within 0.5-mile radius of Well #3 consist of spaced rural residential, single-family residential, railroad right-of-way, and open space park or preserve (County of San Diego 2011).

The parcels on which the Project site is located are zoned as single family residential, undeveloped natural area, open space park and preserve, neighborhood shopping center, and railroad right-of-way. Bordering current land uses to the Project site are open space park and preserve, spaced rural residential, single-family residential, freeway, other retail trade and strip commercial, road right-of-way, airstrip, neighborhood shopping center, and library (County of San Diego 2011) (see Figure 5).

Current land use within the contributing watersheds in Mexico was not available for this report, but is mostly undeveloped lands. Current land use on the United States side of the Flat Creek subwatershed consists of open space park or preserve, field crops, and vacant undeveloped land. Current land use on the United States side of the Boundary Creek subwatershed consists of spaced rural residential, single-family detached, single-family multiple-units, single-family residential without units, communications and utilities, railroad right-of-way, road right-of-way, other retail trade and strip commercial, fire/police station, other public services, elementary school, open space park or preserve, field crops, and vacant and undeveloped land. Current land use on the United States side of the Walker Canyon- Carrizo Creek subwatershed consists of spaced rural residential, single-family detached, single-family multiple-units, single-family residential without units, mobile home park, hotel/motel (low-rise), airstrip, freeway, communication and utilities, railroad right-of-way, road right-of-way, other retail trade and strip commercial, library, post office, religious facility, open space park or preserve, field crops, and vacant and undeveloped land (County of San Diego 2011).

## 2.4	Geology and Soils

### 2.4.1	Geology

Jacumba Hot Springs is located on the eastern portion of the Peninsular Range geomorphic province, which consists of northwest-oriented mountain ranges separated by northwest-trending fault-produced valleys subparallel to faults branching from the San Andreas Fault. The regional geology is depicted in Figure 6, Regional Geologic Map. Because much of the contributing watershed area is located south of the international border with Mexico, worldwide geologic data was used to depict geology south of the border (Garrity and Soller 2009).

The surface area of the contributing watersheds primarily consists of exposed Cretaceous plutonic rocks of the Peninsular Ranges Batholith. These plutonic rocks consist of the bedrock unit known as the tonalite of La Posta (also referred to as the La Posta Quartz Diorite) (USGS 2004). The Sierra Juarez Mountains, located on the southeastern side of the Flat Creek watershed in Mexico consist of Mesozoic sedimentary rocks (Garrity and Soller 2009). Quaternary alluvium is present in low-lying areas in portions of the watershed, including the Jacumba Valley (USGS 2004).

The Project site is located within Jacumba Valley. Jacumba Valley contains exposures of the Jacumba Volcanics and the Table Mountain Formation, overlain by Quaternary alluvium (DWR 2004; Swenson 1981). The Quaternary alluvium reaches up to 175 feet in thickness and consists of Holocene-age gravels, sands, and clays (Dudek 2016a; DWR 2004). The alluvium thins toward the sides and ends of the valley (DWR 2004; Swenson 1981). The Jacumba Volcanics are encountered below the Quaternary alluvium, as reported in numerous boring log reports (County of San Diego 2018; CRA 2012; Petra 2006). The Tertiary-age Table Mountain Formation underlies the Jacumba Volcanics in some areas of Jacumba Valley and is described as medium- to coarse-grained sandstone and conglomerate, and may reach up to 600 feet in thickness (Swenson 1981). The migmatitic schist and gneiss of the Stephenson Peak Formation outcrop just west of the valley and underlie the Jacumba Valley (Swenson 1981; USGS 2004).

### 2.4.2	Soils

The type, areal extent, and key physical and hydrologic characteristics of soils mapped on the United States side of the contributing watersheds were identified based on a review of soil surveys completed by the U.S. Department of Agriculture, Natural Resources Conservation Service (USDA 2015). Swenson (1981) provides a map and description of soil types on the Mexico side of the Flat Creek watershed based on representative soil samples and measurements of their porosity and specific retention. Soils on the Mexico side of the Boundary Creek watershed were digitized based on aerial imagery. Soil units are shown in Figure 7, Soils Map, and are described in Table 2-4.

| Table 2-4 Soil Units within the Contributing Watersheds |  |  |  |
| --- | --- | --- | --- |
| Map Unit, Soil Name | Acres (Percent of the Flat Creek Watershed) | Acres (Percent of the Boundary Creek Watershed | Acres (Percent of the Walker Canyon – Carrizo Creek Watershed) |
| Soil Identification by the U.S. Department of Agriculture |  |  |  |
| AcG, Acid Igneous Rock Land | 0 (0%) | 2,237.66 (15.47%) | 2,105.09 (31.49%) |
| CaB, Calpine coarse sandy loam, 2–5% slope | 0 (0%) | 14.39 (0.10%) | 0 (0%) |
| CaC, Calpine coarse sandy loam, 5–9% slope | 0 (0%) | 14.69 (0.10%) | 264.68 (3.96%) |
| CaD2, Calpine coarse sandy loam, 9–15% slopes, eroded | 0 (0%) | 41.85 (0.29%) | 0 (0%) |
| CeC, Carrizo very gravelly sand, 0–9% slope | 0 (0%) | 796.85 (5.51%) | 0 (0%) |
| InA, Indio silt loam, 0–2% slope | 18.10 (30.33%) | 0 (0%) | 44.90 (0.67%) |
| InB, Indio silt loam, 2–5% slope | 0 (0%) | 0 (0%) | 183.72 (2.75%) |
| IoA, Indio silt loam, saline, 0–2% slope | 0 (0%) | 0.02 (0.0001%) | 382.58 (5.72%) |
| LaE2, La Posta loamy coarse sand, 5–30% slopes, eroded | 0 (0%) | 1,854.48 (12.82%) | 0 (0%) |
| LcE2, La Posta rocky loamy coarse sand, 5–30% slope, eroded | 0 (0%) | 1,649.29 (11.40%) | 43.92 (0.66%) |
| LdE, La Posta-Sheephead complex, 9–30% slopes | 0 (0%) | 2,339.43 (16.17%) | 0 (0%) |
| LdG, La Posta-Sheephead complex, 30–65% slopes | 0 (0%) | 258.21 (1.78%) | 0 (0%) |
| Lu, Loamy alluvial land | 0 (0%) | 17.35 (0.12%) | 0 (0%) |
| MnB, Mecca coarse sandy loam, 2–5% slopes | 4.86 (8.14%) | 0 (0%) | 62.83 (0.94%) |
| MvC, Mottsville loamy coarse sand, 2–9% slopes | 0 (0%) | 948.47 (6.56%) | 0 (0%) |
| MvD, Mottsville loamy coarse sand, 9–15% slopes | 0 (0%) | 65.60 (0.45%) | 0 (0%) |
| RaC, Ramona sandy loam, 5–9% slopes | 0 (0%) | 0 (0%) | 168.35 (2.52%) |
| RaD2, Ramona sandy loam, 9–15% slopes, eroded | 0 (0%) | 0 (0%) | 26.00 (0.39%) |
| RkA, Reiff fine sandy loam, 0-2% slopes | 17.31 (29.00%) | 0 (0%) | 262.87 (3.93%) |
| RsC, Rositas loamy coarse sand, 2–9% slope | 0 (0%) | 152.95 (1.06%) | 531.38 (7.95%) |
| RuG, Rough broken land | 0 (0%) | 0 (0%) | 342.31 (5.12%) |
| SrD, Sloping gullied land | 19.41 (32.53%) | 12.55 (0.09%) | 0 (0%) |
| SvE, Stony land | 0 (0%) | 255.46 (1.77%) | 933.88 (13.97%) |
| ToE2, Tollhouse rocky coarse sandy loam, 5–30% slopes, eroded | 0 (0%) | 3,395.02 (23.47%) | 0 (0%) |
| ToG, Tollhouse rocky coarse sandy loam, 30–65% slopes | 0 (0%) | 413.14 (2.86%) | 0 (0%) |
| Subtotal | 59.68 (0.12%) | 14,467.40 (100%) | 6,685.40 (94.67%) |
| Soil Identification by Swenson |  |  |  |
| W, Sandy Alluvium | 7,020.04 (13.77%) | 0 (0%) | 132.95 (35.35%) |
| X, Metamorphic and Plutonic Residuum | 43,462.93 (85.27%) | 0 (0%) | 93.11 (24.76%) |
| Y, Volcanic residuum and fine sand alluvium | 489.09 (0.96%) | 0 (0%) | 150.04 (39.89%) |
| Subtotal | 50,972.06 (99.88%) | 0 (0%) | 376.10 (5.33%) |
| Total Acreage | 51,031.73 | 14,467.40 | 7,061.50 |

Sources: Swenson 1981; USDA 2015

## 2.5	Hydrogeologic Units

The Project site is located within the California Department of Water Resource’s (DWR) Bulletin 118 defined Jacumba Valley Groundwater Basin, DWR Basin No. 7-47 (Figure 8, Hydrogeologic Units) (DWR 2004). The Jacumba Valley Groundwater Basin consists of two primary aquifer units. The upper alluvial aquifer unit reaches up to 175 feet in thickness and consists of Holocene-age gravels, sands, and clays (Dudek 2016a; DWR 2004). In some areas, this aquifer unit is underlain by the Jacumba Volcanics that act as a semi-confining to confining unit to the lower aquifer. The lower aquifer consists of the Tertiary-age Table Mountain Formation described as medium- to coarse-grained sandstone and conglomerate, and may reach up to 600 feet in thickness (Swenson 1981). The Table Mountain Formation lies unconformably on top of crystalline basement (DWR 2004).

On-site Project groundwater wells produce from the upper alluvial aquifer (referred to in this report as the “Jacumba Valley alluvial aquifer”). This unconfined aquifer has been estimated to have specific yields ranging from 5% to 10% (Swenson 1981) and 15% to 20% (Roff and Franzone 1994). Production wells screened in the Jacumba Valley alluvial aquifer have been reported to produce more than 1,000 gpm (Roff and Franzone 1994). Groundwater in storage has been estimated to range from 3,200 to 6,400 acre-feet by Swenson (1981) and 9,600 to 16,000 acre-feet by Roff and Franzone (1994).

## 2.6	Current Groundwater Demand

The current water demand for the Jacumba Valley alluvial aquifer includes potable demand for Jacumba Valley Ranch Water Company (formerly the Ketchum Ranch Water Company), and potable and non-potable demand from the Jacumba Community Services District (JCSD) (Table 2-5).

The Jacumba Valley Ranch Water Company is classified as a transient non-community water system. According to County Department of Environmental Health Small Drinking Water System files, seven connections—three ranch homes, two gas stations, and two fire hydrants—are part of the Jacumba Valley Ranch water system (McCullough, pers. comm. 2015). Estimated water demands for the Jacumba Valley Ranch Water Company is 5 afy.

JCSD currently supplies potable water to 239 connections from JCSD Well #4 (Devine, pers. comm. 2019). JCSD’s current water usage was not made available for this report, but historical water demand and water use calculations were used to estimate current demand. Based on available data from Barrett Consulting Group (Barrett 1996), JCSD produced between 86 and 146 acre-feet annually from 1991 to 1995, averaging 116 afy. More recent production data indicates that JCSD served 27.6 million gallons (85 acre-feet) of water from Well #4 in 2013 and 26.2 million gallons (80.4 acre-feet) from January 2014 through August 2014 to meet the water demands of the potable water system (Troutt, pers. comm. 2015). Based on the number of connections and an estimated 0.5 afy per connection, JCSD potable water demand is estimated to be 119.5 afy. This estimate roughly coincides with average historical water demand from 1991 to 1995, and conservatively overestimates production from more recent data received by the previous JCSD General Manager in 2014 (Troutt, pers. comm. 2015).

JCSD also supplies non-potable water for commercial sale. Historically, JCSD has supplied non-potable water from Well #6, a fractured rock well not screened in the Jacumba Valley alluvium. Beginning in 2016, JCSD began supplying non-potable water from the Highland Center Well and the Park Well, both screened in the Jacumba Valley alluvium. Non-potable water supply from JCSD varied based on customer demand. From February 2017 to February 2018, JCSD supplied 50.1 acre-feet from the Highland Center Well and 3.5 acre-feet from the Park Well. From February 2018 to January 2019, JCSD supplied 4 acre-feet from the Highland Center Well and 0 acre-feet from the Park Well. Maximum annual groundwater extraction from the Jacumba Valley alluvial aquifer by JCSD for non-potable water is 53.6 afy.

Based on the County Department of Environmental Health well completion report database, no additional active wells are located within the Jacumba Valley alluvium (County of San Diego 2018). Because there is the potential for active wells to exist without proper County Department of Environmental Health permitting, this report conservatively estimates six potential domestic wells that produce groundwater from the Jacumba Valley alluvial aquifer. Estimated water demands for the potential domestic wells is 3 afy, or 0.5 afy per well.

Agriculture located on the Jacumba Valley Ranch historically extracted the majority of groundwater from the Jacumba Valley alluvial aquifer. Currently no water is being extracted from the Jacumba Valley Ranch for these activities.

| Table 2-5 Jacumba Valley Alluvial Aquifer Existing Water Demands |  |  |
| --- | --- | --- |
| Groundwater Extraction Sources | Wells Names | Total Water Demand (acre-feet per year) |
| Jacumba Valley Ranch Water Co. | Well Km | 5a |
| Jacumba Community Services District (JCSD) (potable) | Well 4 | 119.5b |
| JCSD (non-potable) | Highland Center Well, Park Well | 53.6c |
| Potential Domestic Wells | Private Domestic Wells | 3d |
| Total Water Demand |  | 181.1 |

- Jacumba Valley Ranch Water Company has seven connections: three ranch homes, two gas stations, and two fire hydrants. No water demand was assigned to the fire hydrants. Water demand is estimated at approximately 1 acre-foot per connection.

- Estimated based on 0.5 afy for 239 potable Jacumba Community Services District connections.

- Maximum demand based on meter reads from February 2017 to February 2018.

- Not all domestic wells are currently active or known; however, a consumptive water demand of 0.5 afy has been assigned to up to six potential domestic wells

## 2.7	Hydrogeologic Inventory and Groundwater Level Trends

Published well logs were reviewed to locate wells and refine the thickness of hydrologic units present within the Jacumba Valley alluvial aquifer. Table 2-6 provides a summary of the information available from driller well logs obtained to date. Well information has been updated based on field reconnaissance and/or historical data

| Table 2-6 Jacumba Valley Well Inventory |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Well Number | Well Depth (feet bgs)/ (Year Drilled) | Depth to Water (feet btoc)/date | Approximate Production Capability (gpm) | Alluvium/ Residual Soil (feet bgs) | Bedrock Depth (feet bgs)/ (Type) |
| Jacumba Community Services District Wells |  |  |  |  |  |
| JCSD 1 | 124 (1956) | 43.0; 10/1955 | 148 | 120 | 124 (volcanic) |
| JCSD 2 | 140 (1963) | 72.13; 11/1979 | — | 140 | — |
| JCSD 3 | 79 | — | — | — | — |
| JCSD 3A | 49 | — | — | 49 | — |
| JCSD 4 | 39 | 20.66; 6/26/2018 | 175a | 0–39b | — |
| JCSD 5 | — | — | — | — | — |
| JCSD 6 | 465 (2003) | 5.50; 6/26/2018 | 600+ | — | — |
| JCSD 7 | 518 (2008) | 31.20; 6/26/2018 | 300+ | 0–10 | 10–23 (granitic) |
| JCSD 8 | 518 (2009) | 31.02; 6/26/2018 | 275+ | 0–42 | 42–55 (granitic) |
| MW-3 | 84.5 (2007) | 28.0; 3/2009 | Monitor well | 0–30 | 30–80 (granitic) |
| Park Well | 124 (2005) | 59.74; 6/26/2018 | 80 | 0–127 | 127 (volcanic) |
| Highland Center Well | 125 (2016) | 56.98; 6/26/2018 | 174 | 0–175 | 182 (granitic) |
| Jacumba Valley Ranch Wells |  |  |  |  |  |
| K | 102+ (1960s) | — | — | — | — |
| K1 | 110 (1950s) | 42.3; 9/6/1980 | — | 106 | — |
| K2 | 103 (1950s) | 41.0; 4/1958 | — | 103 | — |
| K3 | 117 (1950s) | 8.5; 2/1996 | 1,000 | — | — |
| K4 | 109 (1950s) | 9.9; 3/1994 | 908 | — | — |
| Daley Well | 150 (Unknown) | 36.94; 10/2018 | — | — | — |
| Well #1 | 124 (Unknown)d | 59.99; 10/2018 | 148 | 120 | 124 (volcanic) |
| Well #2 | 114 (2007)d | 46.56; 10/2018 | 2,000c | 113 | — |
| Well #3 | 100 (2005)d | 38.96; 10/2018 | 2,000c | 112 | — |
| Central Irrigation Well | 100 (Unknown)d | 46.56; 10/2018 | — | — | — |
| Mid Valley Well | 90.7 (Unknown)d | 48.72; 10/2018 | — | — | — |
| Carrizo Gorge Well | — | 80.22; 7/2018 | — | — | — |
| Well Km | 150 (130 silted) | 51.62; 7/2018 | 33.7 | — | — |
| Test Well 1 JVR | 82 (1990) | 2; 5/1990 | 225 | 75 | — |
| P-1 | — | — | Monitoring well | — | — |
| P-2 | 23.72d | Dry; 7/30/2018 | Monitoring well | — | — |
| P-3 | 30.92d | Dry; 7/30/2018 | Monitoring well | — | — |
| P-4 | 33.71d | Dry; 7/30/2018 | Monitoring well | — | — |
| P-5 | 27.3d | Dry; 7/30/2018 | Monitoring well | — | — |
| P-6 | 32.26d | Dry; 7/30/2018 | Monitoring well | — | — |
| P-7 | 38.8d | Dry; 7/30/2018 | Monitoring well | — | — |
| P-8 | 39.3d | Dry; 7/30/2018 | Monitoring well | — | — |
| P-9 | 60.17d | Dry; 7/30/2018 | Monitoring well | — | — |
| Other Wells |  |  |  |  |  |
| R1 | 137 | — | — | — | — |
| R2 | 400 | — | — | — | — |
| (Abandoned Well near R2) | Abandoned (1979) | — | — | — | 150–492 (Sandstone) |
| T5 | — | — | — | — | — |
| T8 | — | — | — | — | — |
| T1 | — | — | — | — | — |
| RM | 34 | — | — | — | — |
| Spa Well | 200 (1955) | — | — | — | — |
| Daley Construction Well | 230 (NA) | — | — | — | — |
| Former Chevron Service Station 20-5934 |  |  |  |  |  |
| MW-8S | 50 (2007) | — | — | 81.5+ | — |
| MW8-D | 80 (2007) | — | — | 81.5+ | — |
| MW-9S | 50 (2007) | — | — | 80 | 80 (Volcanics) |
| MW-9D | 80 (2007) | — | — | 80 | 80 (Volcanics) |
| MW-10 | 57 (2007) | — | — | 50+ | — |
| MW-11 | 80 (2007) | — | — | 80+ | — |
| MW-12 | 80 (2012) | — | — | 40 | 40 (DG to 80.5) |
| MW-13 | 80 (2012) | — | — | 81+ | — |
| MW-14 | 81 (2012) | — | — | 80.5+ | — |
| B-10 | (2012) | — | — | 55.5+ | — |
| B-11 | (2012) | — | — | 66.5+ | — |
| B-12 | (2012) | — | — | 57 | 57 (DG to 70) |

Sources: Barrett 1996; Pape 2015; Petra 2006; Swenson 1981; GRA 2012

bgs = below ground surface; btoc = below top of casing; gpm = gallons per minute; JCSD = Jacumba Community Services District; NA = not available; DG = decomposed granite

- Reported pumping capacity provided by JCSD.

- Alluvial depth based on total depth of Well #4.

- Pumping rate based on airlifting by driller.

- Based on field reconnaissance conducted in 2018 by Dudek staff.

Groundwater level data were obtained from JCSD from January 2012 through June 2018 (Devine, pers. comm. 2019; Troutt, pers. comm. 2015). Groundwater level data were also obtained from Barrett Consulting Group (1996), Peterson (2014), and Swenson (1981). Historical groundwater level data were available for Jacumba Valley as far back as 1955, but a continuous water level record was not available. On-site groundwater levels were recently measured by Dudek in July, October, and December 2018.

Fluctuations in water levels in the Jacumba Valley alluvial aquifer result from both groundwater production and cycles of wet and dry climatic periods. Historical groundwater measurements from wells K1, K2, and K3 were used to represent trends associated with previous land use on the Project site (Exhibit 2-2). Wells K1, K2, and K3 have the closest geographical relationship to the Central Irrigation Well, Mid Valley Well, and Well #2, respectively.

Exhibit 2-2
Jacumba Valley Alluvial Aquifer Groundwater Level Data July 1955 to December 2018

Sources: Barrett 1996; Pape 2015; Peterson 2014; Swenson 1981.

Note: Boxes outlined by dashes represent wells in similar geographical locations.

Groundwater levels have fluctuated up to 61 feet in Well K3. When Well K3 was initially drilled in 1955, the groundwater level was 38.5 feet below ground surface (bgs). From 1932 to 1977, Jacumba Valley Ranch extracted on average 2,066 afy from the Jacumba Valley alluvial aquifer (Barrett 1996). Jacumba Valley Ranch pumping, in combination with lower than average precipitation in the late 1960s through the mid-1970s (see declining cumulative departure from mean precipitation in Exhibit 2-1), resulted in a groundwater level decline in the Jacumba Valley alluvial aquifer (Exhibit 2-2). Irrigation of agricultural lands ceased on Jacumba Valley Ranch in approximately 1977. In 1979, the groundwater level in Well K3 was 69.9 feet bgs (more than 30 feet lower than initial water level recorded in 1955). By 1990, groundwater levels had risen to near the surface in several Jacumba Valley alluvial aquifer wells (9 feet bgs in Well K3) because of higher recharge rates during a period of above-average precipitation in the late 1970s to mid-1980s (see ascending cumulative departure from mean precipitation in Exhibit 2-1) and low groundwater extraction during this time period.

Groundwater levels from the Central Irrigation Well declined from 2006 to 2011. This decline coincided with a lower than average rainfall period from 1999 to 2008 and the extraction of approximately 741 afy of groundwater by Bornt Farms. Groundwater levels began to rise after Bornt Farms ceased groundwater extraction in 2013. The current gradual declining trend in groundwater levels, shown in Well #2, can be attributed to lower than average rainfall years and recent extraction from JCSD non-potable wells. The groundwater level in Well #2 is currently 11.9 feet above the historic low groundwater level observed in Well K3, located near Well #2.

## 2.8	Water Quality

Spring water in the northern area of the Jacumba Valley at Carrizo Gorge had measured total dissolved solids concentrations ranging from 2,000 to 6,000 milligrams per liter. Surface water drainage measured from the Flat Creek watershed and the Boundary Creek watershed have had recorded total dissolved solids concentrations at 292 to 422 milligrams per liter and 1,640 milligrams per liter, respectively (Roff and Franzone 1994). Historically, groundwater included sodium chloride, calcium chloride, and calcium sulfate (Roff and Franzone 1994).

JCSD supplies non-potable water from the Park and Highland Center Wells, and potable water from Well #4. A water quality sample collected from the Highland Center Well in 2016 had a measured total dissolved solids concentration of 400 milligrams per liter. A wide range of constituents, including general minerals, inorganic minerals, and volatile organic compounds, were analyzed. Laboratory results indicated that no volatile organic compounds were detected and that groundwater produced from the Highland Center Well is suitable for construction water supply (Dudek 2016a). The Park Well was initially intended for use as a potable water well; however, low concentrations of volatile organic compounds were detected during drilling. Toluene was detected at concentrations of 291 micrograms per liter (µg/L), 199 µg/L, and 520 µg/L in water quality samples collected from the Park Well in 2006 (Petra 2006). A subsequent water quality sample was collected from the Park Well on November 5, 2015, by Dudek staff. Results from the sample collected on November 5, 2015, indicated no detections above the reporting limits for all constituents analyzed, including toluene, which was previously detected in the Park Well above the drinking water maximum contaminant level of 150 µg/L. It is possible that the toluene was introduced into the Park Well as a result of drilling or from chemicals (Scothchkote™) used in splicing the submersible cable for installation of the submersible pump and motor when the well was originally tested. Dudek has previously detected toluene in other water wells after the use of Scothchkote (EnviroMatrix Analytical 2015).

Since the Project would use groundwater for non-potable use, water quality samples were not collected from on-site wells.

INTENTIONALLY LEFT BLANK

# 3	Water Quantity Impacts Analysis

This section discusses the potential impacts on local groundwater resources in terms of the County Guidelines (County of San Diego 2007).

## 3.1	50% Reduction of Groundwater Storage

To apply the County methodology for determining a 50% reduction in groundwater storage to a given well, the area of the aquifer that can be accessed by a pumping well must be defined. For this analysis, the 2,061-acre extent and variable thickness of the alluvium underlying the Jacumba Valley as defined by Swenson (1981) was used to perform the 50% reduction in storage analysis.

### 3.1.1	Guidelines for Determination of Significance

The following requirement is set forth in the County Guidelines (County of San Diego 2007):

For proposed projects in fractured rock and sedimentary basins, groundwater impacts will be considered significant if a soil moisture balance, or equivalent analysis, conducted using a minimum of 30 years of precipitation data, including drought periods, concludes that at any time groundwater in storage is reduced to a level of 50% or less as a result of groundwater extraction.

A Project-specific soil-moisture-based water balance was not performed for the Project site. Instead, an updated estimate of groundwater in storage was made based on previous work conducted by Roff and Fanzone (1994) and Swenson (1981). The estimate evaluated whether the water demands for the Project would maintain at least 50% groundwater in storage over the 2,061-acre Jacumba Valley alluvial aquifer (mapped by Swenson 1981). Additionally, a one-time Project extraction of up to 112 acre-feet over a 1-year period was compared to historical groundwater extraction rates from the Jacumba Valley alluvial aquifer.

### 3.1.2	Methodology

#### 3.1.2.1 	Groundwater Recharge

Groundwater recharge was not calculated for the contributing watersheds or the Jacumba Valley alluvial aquifer.

#### 3.1.2.2 	Groundwater Demand

Historical Demand

The groundwater demands of the Jacumba Valley alluvial aquifer vary with time. Historically, Jacumba Valley Ranch was the primary user of groundwater from the aquifer. Jacumba Valley Ranch produced water for irrigation of agricultural lands. From 1932 through 1977, Jacumba Valley Ranch extracted on average 2,066 afy of groundwater (Barrett 1996). Irrigation ceased on Jacumba Valley Ranch and the agricultural lands were fallowed from about 1977 until 2002. From 2002 until 2013, Bornt Farms resumed irrigation at Jacumba Valley Ranch. The water demand of Bornt Farms was reported to be in excess of 1 million gallons per day (Pape, pers. comm. 2015). To determine the area of active irrigated agricultural land by year, historical aerial photographs were reviewed. Between 2002 and 2013, 187 to 465 acres of the Jacumba Valley Ranch was irrigated to grow predominantly lettuce and spinach (Google Earth 2015). Assuming a crop irrigation rate of 2.14 acre-feet per acre for lettuce, the maximum annual water demand of the lettuce crop at Bornt Farms would be 995 acre-feet (Barrett 1996; U.C. Davis 2011). Other estimates state that Bornt Farms extracted 7,413 acre-feet over the farm’s lifetime, or an average of 741.3 afy.

Other groundwater users include the Jacumba Valley Ranch Water Company, which has historically extracted in excess of 242 afy (Barrett 1996). Groundwater extraction on the Mexican side of the border has historically been estimated to be 24 afy (Barrett 1996).

Since 1985, JCSD has extracted potable water from up to four groundwater wells within its approximately 423-acre boundary (LAFCO 2013). The water system includes storage of up to 638,000 gallons. As discussed in Section 2.6, Current Groundwater Demand, historical potable water demand has been documented to be between 85 and 146 afy (Barret 1996; Trout, pers. comm. 2015).

As discussed in Section 2.6, JCSD has historically supplied non-potable water for commercial sale from Well #6 (a fractured rock well not screened in the Jacumba Valley alluvium) and the Highland Center Well and Park Well (both screened in the Jacumba Valley alluvium). Non-potable water supply from JCSD varies based on customer demand. Based on meter reads by Dudek staff, from February 2017 to February 2018, JCSD supplied 50.1 acre-feet from the Highland Center Well and 3.5 acre-feet from the Park Well. Maximum annual groundwater extraction from the Jacumba Valley alluvial aquifer by JCSD for non-potable water is 53.6 afy.

Current Demand

Current groundwater demand from the Jacumba Valley alluvial aquifer includes extraction by JCSD, Jacumba Valley Ranch Water Company, and a few potential domestic well owners. The Project site, which was historically produced an excess of 2,000 afy, no long extracts groundwater for agriculture. The Jacumba Valley Ranch Water Company, which has historically extracted an excess of 242 afy, currently supplies approximately 5 afy for three ranch homes, two gas stations, and two fire hydrants (Barrett 1996; McCullough, pers. comm. 2015).

JCSD continues to extract both potable and non-potable groundwater from the Jacumba Valley alluvial aquifer. As discussed in Section 2.6, JCSD is estimated to produce approximately 119.5 afy of potable water for 239 connections from Well #4, and 4 afy of non-potable water during 2018 from the Highland Center Well and Park Well (Devine, pers. comm. 2019).

There may be small volumes of groundwater (less than 3 afy) extracted from domestic wells located in the residential area in Jacumba Hot Springs.

Groundwater extraction is occurring from the fractured rock aquifer by JCSD, Jacumba Hot Springs Resort, and a few domestic well users on the outskirts of town. Since the Project is proposing to extract groundwater from the Jacumba Valley alluvial aquifer, groundwater extraction from the fractured rock aquifer was not included in this analysis.

Future Demand

Future demand is expected to include JCSD potable and non-potable demand, Jacumba Valley Ranch Water Company, and private domestic users. Potable groundwater use from JCSD, the Jacumba Valley Ranch Water Company, and private domestic users is expected to be similar to current conditions over the long-term. JCSD has the potential to serve non-potable from the Highland Center and the Park Well.

Based on the current pump capacity of the Highland Center and Park Wells, maximum non-potable annual production from JCSD wells screened in the Jacumba Valley alluvial aquifer is 410 afy. This estimate is based on continuous pumping for 1 year at a maximum flow rate of 174 gpm and 80 gpm from the Highland Center Well and Park Well, respectively.

JCSD is currently completing a manganese water treatment system for Wells #7 and #8 that will ultimately serve all potable water demands for its customers (Dudek 2016b). This treatment system is expected to come online in the next few months (spring 2019). Once the treatment facility is operational, the JCSD water supply will be sourced from the fractured rock aquifer rather than the Jacumba Valley alluvial aquifer.

Table 3-1 provides historical, current, and future water demand from the proposed Project, other proposed projects, and Project O&M. The future projected water demand conservatively evaluates the Project and other proposed projects taking place concurrently.

| Table 3-1 Jacumba Valley Alluvial Aquifer Groundwater Demand |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Land Use | Historical Water Demand (afy) | Current Water Demand (afy) | Future Water Demand During Construction (afy) | Future Ongoing Water Demand (afy) | Future Maximum Demand (afy) |
| Project Site (Jacumba Valley Ranch; Bornt Farms;) | 2,066; 741–995 | 0 | 112 | 10 | 112 |
| Jacumba Valley Ranch Water Company | 242 | 5 | 5 | 5 | 5 |
| Private Domestica | 3 | 3 | 3 | 3 | 3 |
| JCSD (Potable) | 80–146b | 119.5 | 0c | 0c | 119.5 |
| JCSD (Non-Potable) | 53.6 | 4d | 4d | 4d | 410e |
| Total Estimated Water Demand | 2,212f | 131.5 | 12 | 12 | 537.5 |
| Total Estimated Water Demand With Project | 2,212f | 131.5 | 124 | 22 | 649.5 |

Source: Barrett 1996; Dudek 2015; Troutt, pers. comm. 2015

afy = acre-feet per year; JCSD = Jacumba Community Services District

- Not all domestic wells are currently active or known; however, a consumptive water demand of 0.5 afy has been assigned to up to six potential domestic wells

- JCSD Wells #1 and #2 supplied all potable demands for the town of Jacumba Hot Springs until JCSD Wells #3 and #4 were drilled in the early 1970s.

- Future JCSD potable water demand will be supplied from Wells #7 and #8, completed in the fractured rock aquifer.

- Assumes current groundwater demand based on Dudek metered data from 2018.

- Assumes maximum groundwater extraction based on tested well yields from the Highland Center Well and the Park Well.

- Assumes maximum concurrent water demand from JCSD potable demand and Jacumba Valley Ranch.

Historically, groundwater demand from the Jacumba Valley alluvial aquifer has been estimated to be up to 2,066 afy (Barrett 1996). A drastic reduction in groundwater production has occurred since agriculture irrigation ceased on Jacumba Valley Ranch. The current groundwater demand from the Jacumba Valley alluvial aquifer is estimated to be 131.5 afy (Table 3-1). An additional 112 acre-feet would be extracted during Project construction, resulting in a 1-year extraction of 243.5 acre-feet from the aquifer, assuming other groundwater users continue their current estimated extraction amounts. However, starting in spring 2019, groundwater extraction from the Jacumba Valley alluvial aquifer for JCSD potable use is expected to cease after the completion of a manganese water treatment system for fractured rock Wells #7 and #8. This will result in a reduction of water demand from the Jacumba Valley alluvial aquifer, taking into account water demand for Project construction. The total water demand from the Jacumba Valley alluvial aquifer during Project construction is expected to be 124 acre-feet, which includes the Project and ongoing use, minus JCSD potable demand.

After Project construction, ongoing groundwater productions from the alluvial aquifer is estimated to be 22 afy, based on 10 afy of Project water use for O&M and 4 acre-feet of continuous non-potable water use by JCSD (Table 3-1). Additionally, the Project would extract groundwater for decommissioning in the future.

To account for the potential for JCSD to supply non-potable water to available customer, an additional water demand analysis was complete. This future maximum demand analysis assumes that non-potable groundwater is extracted by the JCSD at the maximum pump capacity of the Highland Center and Park Well. The total projected groundwater extraction for ongoing water use, the proposed Project, and maximum demand from non-potable JCSD wells is 649.5 afy.

#### 3.1.2.3	Groundwater in Storage

Groundwater in storage was calculated using estimates of the saturated aquifer thickness underlying the 2,060-acre area of the Jacumba Valley alluvial aquifer, as mapped by Swenson (1981). Aquifer thickness was updated from the Swenson groundwater storage compartments (A through E) with available well completion information. The estimated saturated thickness is based on recent groundwater levels measured in June and December 2018. The updated well completion information used to constrain aquifer thickness is provided in Table 3-2 and included in Appendix A, Well Completion Information. For compartments with multiple wells and groundwater level measurements, values were averaged to represent a non-uniform saturated aquifer thickness. In all cases, the average saturated thickness used to define groundwater in storage (Table 3-3) was less than the measured saturated thickness at each well (Table 3-2). For compartments in which no wells were located, groundwater levels were extrapolated from the nearest well (Table 3-3). Groundwater storage compartments and their representative wells are depicted in Figure 8. Specific yield was estimated based on historical and recent aquifer test analyses.

| Table 3-2 Well Completion Information for Constraining Alluvial Saturated Thickness |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Common Well Name | Source or County of San Diego Well Record Identification | Aquifer Thickness (feet) | Depth to Groundwater/ (feet below ground surface) | Depth to Groundwater Measurement Date | Saturated Thickness (feet) | Swenson Compartment (Swenson 1981) |
| JVR – Carrizo Creek | Lwell 6933 | 55 | — | — | — | A |
| Leighton B-12 | Leighton 1991a | 20 | — | — | — | A |
| Well #3 | Lwel 16419 | 89 | 35.14 | 12/11/2018 | 50.26 | C |
| Well #2 | Lwel 1815 | 113 | 56.21 | 12/11/2018 | 55.27 | C |
| Test Hole | Lwel 20450 | 100 | — | — | — | C |
| Leighton B-2 | Leighton 1991a | 25 | — | — | — | C |
| Central Irrigation Well | — | — | 44.33 | 12/11/2018 | — | C |
| Mid-Valley Well | — | — | 47.42 | 12/11/2018 | — | C |
| Well #1 | — | 124 | 57.87 | 12/11/2018 | — | D |
| J2 | Swenson 1981 | 120 | — | — |  | D |
| Test Hole | Lwel 17922 | 108 | — | — | — | D |
| Southwest Irrigation | Lwel 18031 | 86 | — | — | — | D |
| Test Hole | Lwel 20411 | 150 | — | — | — | D |
| Highland Center Well | Lwel 001506 | 175 | 56.98 | 6/26/2018 | 118.02 | E |
| Park Well | — | — | 59.74 | 6/26/2018 | — | E |
| J3 | Swenson 1981 | 60 | — | — | — | E |
| J4 | Swenson 1981 | 50 | — | — | — | E |

— = no information is available

Specific Yield (Storage Coefficient)

Previous estimates of specific yield for the Jacumaba Valley alluvial aquifer were made by Swenson (1981) and calculated from aquifer testing performed by Barrett (1996). The specific yield associated with the alluvium was conservatively estimated by Swenson (1981) to be between 5% and 10%. Barrett (1996) estimated specific yield to be 25% based on aquifer testing of Well K4, Test Well No. 1, and Well Km.

Storativity (storage coefficient) was calculated for this report (Section 3.2) based on two constant-rate aquifer tests. The storage coefficient from the Well #2 aquifer test, located in compartment D, ranged from 0.008 to 0.028. The storage coefficient from the Well #3 aquifer test, located in compartment C, was calculated to be 0.2349 (Geosyntec 2012). Since the aquifer tests were conducted in the unconfined aquifer, the calculated storage coefficient is equivalent to the specific yield (Driscoll 1986). Values for the storage coefficient for unconfined aquifers range from 0.01 to 3 (Driscoll 1986). The calculated storage coefficients from the Well #2 and Well #3 aquifer tests fall within this range.

Based on recent aquifer test analysis performed on Well #2 and Well #3 within the Jacumba Valley alluvial aquifer, the specific yield ranges from 0.08% to 24%, with a mean value of 12% (Geosyntec 2012). To provide a conservative estimate, a specific yield value of 10% was used for this analysis to calculate groundwater in storage.

Saturated thickness was calculated by subtracting the average alluvial thickness by recent depth to groundwater measurements recorded in 2018. Saturated thickness for each compartment was then multiplied by the compartments acreage and the 10% specific yield value to determine the groundwater in storage by compartment. Based on these calculations, the current groundwater in storage within the Jacumba Valley alluvial aquifer is estimated to be 9,005 acre-feet (Table 3-3).

In comparison, groundwater in storage was estimated to range from 9,600 to 16,000 acre-feet by Roff and Fanzone (1994), and from 3,200 to 6,400 acre-feet by Swenson (1981).

The Project proposes to use 112 acre-feet during construction for 1 year. Assuming no recharge to the aquifer, the Project alone would reduce groundwater in storage by 1.2% during construction, which is substantially less than the 50% reduction in storage criteria. The estimated maximum extraction from all known sources during the period of Project construction is 649.5 acre-feet. Total reduction of groundwater in storage from all sources during the construction period is estimated to be 7.2%. Assuming a Project lifetime of 40 years (1 year of construction, 38 years of O&M, and 1 year of decommissioning), the proposed Project would use 542 acre-feet of water. This equates to a 6% reduction in storage from Project groundwater extraction over 40 years, assuming no recharge to the aquifer.

| Table 3-3 Jacumba Valley Alluvial Aquifer 2018 Groundwater in Storage Estimate |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Alluvial Aquifer Compartments* | Area (acres) | Leighton Alluvial Thickness (1991) (feet) | Average Alluvial Thickness (feet) | Depth to Water 2018 (feet bgs) | Average Saturated Thickness (feet) | Specific Yield (unitless) | Storage (acre-feet) |
| A | 240.94 | 50+ | 37.5 | 35.14 | 2.36 | 0.10 | 56.86 |
| B | 104.70 | 50+ | 50 | 35.14 | 14.86 | 0.10 | 155.58 |
| C | 439.40 | 120+ | 81.75 | 43.5 | 38.25 | 0.10 | 1,680.71 |
| D | 1,082.73 | 100+ | 117 | 57.87 | 59.13 | 0.10 | 6,402.18 |
| E | 193.61 | 80+ | 95.0 | 58.36 | 36.64 | 0.10 | 709.39 |
| Total Groundwater in Storage (rounded acre-feet) |  |  |  |  |  |  | 9,005 |

* 	Compartment Details:

A	Aquifer thickness estimated from an average alluvial thickness observed in well log Lwel 6933 and B-12 (Leighton 1991a). Depth to water extrapolated from Well #3 (Lwel 16419)

B	Aquifer thickness defined by Leighton 1991a. Depth to water extrapolated from Well #3 (Lwel 16419)

C	Aquifer thickness estimated from Well #3 (Lwel 16419), Well #2 (Lwel 1814), Test Hole (L well 20450), and Leighton B-7 (Leighton 1991a). Depth to water averaged from Well #3 (Lwel 16419) and Well #2 (Lwel 1814).

D	Aquifer thickness estimated from Well J2 (Swenson 1981), Test Holes (Lwell 17922 and 201411), and the Southwest Irrigation Well (Lwell 18031). Depth to water estimated from Well #1.

E	Aquifer thickness estimated from the Highland Center Well (Lwell 001506), and Wells J3 and J4 (Swenson 1981). Depth to water estimated from an average of the Highland Center Well (Lwell 001506) and the Park Well.

#### 3.1.2.4	Long-Term Groundwater Availability (Sustainability)

Long-term groundwater availability was evaluated in context of the current available groundwater in storage, historical groundwater levels, and water demand. The volume of groundwater in storage varies depending on the rate of recharge and the volume of water pumped from storage (water demand). Sustainable groundwater availability is less than the historical average groundwater production rate of 2,066 afy from 1932 to 1977. This is observed during dry periods when the Jacumba Valley experienced groundwater overdraft, as indicated by declining groundwater levels in the alluvial aquifer wells (Exhibit 2-2). Pumping by Jacumba Valley Ranch between 2003 and 2013 also resulted in groundwater level declines in the alluvial aquifer. Bornt Farms grew lettuce and spinach on up to 465 acres, year-round, with an estimated maximum extraction rate of 995 acre-feet per year (Barrett 1996; UC Davis 2011). Due to Bornt Farms irrigation and below-average precipitation recorded in the contributing watersheds over the last decade, the water demands exceeded available recharge, resulting in groundwater level decline (Exhibit 2-2). Several years of drought and limited non-potable extraction by JCSD likely contributed to the current groundwater level decline.

The Project proposes to extract groundwater for 1 year at a maximum quantity of 112 acre-feet. This one time use of groundwater for construction is approximately 10% of the annual production quantity of Bornt Farms, and 5% of the annual production quantity of Jacumba Valley Ranch. After Project construction, groundwater extraction for O&M would be 0.9% of the annual production quantity of Bornt Farms and 0.5% of the annual production quantity of Jacumba Valley Ranch for the maximum groundwater historically extracted from the Project site. Groundwater extraction for decommissioning and dismantling would be 5% of the annual production quantity of Bornt Farms and 2% of the annual production quantity of Jacumba Valley Ranch for the maximum groundwater historically extracted from the Project site.

### 3.1.3	Significance of Impacts Prior to Mitigation

The results of the analysis show that historical groundwater extraction rates of 995 to 2,066 afy resulted in groundwater overdraft during dry climatic periods such as those experienced from 1963 to 1976, and 1998 through 2008. Between 1955 and 1978, in conjunction with high pumping rates and low recharge rates, groundwater levels in the Jacumba Valley alluvial aquifer decreased by approximately 30 feet. The groundwater overdraft and storage reduction observed in the Jacumba Valley alluvial aquifer between 1938 and 1978 was alleviated, however, by 1993 when groundwater levels recovered to within 8 feet of land surface at Well K3 (Exhibit 2-2). This data shows that aquifer recharge is as important as groundwater withdrawal for maintaining adequate storage in the aquifer. The groundwater extraction for Project construction, a one-time use of up to 112 afy, is 10% of the historical average groundwater extraction at the Project site and 5% of the total maximum production from the entire Jacumba Valley alluvial aquifer. The total estimated Project groundwater extraction for the 40-year lifetime (542 acre-feet) of the Project is 6% of the current groundwater in storage.

### 3.1.4	Mitigation Measures and Design Considerations

Actual conditions during groundwater extraction for the Project may vary from the above analysis, so a Groundwater Monitoring and Mitigation Plan (GMMP) will be prepared to ensure that pumping does not significantly impact existing well users. The GMMP will provide for monitoring the duration and rate of Project pumping to document the total volume of groundwater extracted. The GMMP will also provide for monitoring groundwater levels from Project pumping and monitoring wells.

### 3.1.5	Conclusions

The proposed Project would have a less-than-significant impact to groundwater in storage, as defined by the County Guidelines (County of San Diego 2007). Project groundwater extraction for construction would equate to a 1.2% reduction in groundwater storage. Total Project groundwater extraction for the life of the Project, assuming a 40-year lifespan, would equate to 6% of current groundwater in storage. Both values are far less than the County’s significance criteria of 50%.

Total groundwater extraction from the Jacumba Valley alluvial aquifer, including the Project and all proposed projects (maximum non-potable extraction from non-potable JCSD wells), would reduce groundwater in storage by 7.2%, and would be 31% of the historical maximum groundwater extraction.

## 3.2	Well Interference and Groundwater Dependent Habitat

### 3.2.1	Guidelines for Determination of Significance

#### 3.2.1.1	Well Interference

The following significant impact requirements are set forth in the County Guidelines (County of San Diego 2007):

Alluvial Well: As an initial screening tool, off-site well interference will be considered a significant impact if after a five year projection of drawdown, the results indicate a decrease in water level of 5 feet or more in the off-site wells. If site-specific data indicates alluvium or sedimentary rocks exist which substantiate a saturated thickness greater than 100 feet in off-site wells, a decrease in saturated thickness of 5% or more in the off-site wells would be considered a significant impact.

According to the County Groundwater Geologist, the primary author of the County of San Diego Guidelines, the intent of the above guideline was to cover projects that have continual ongoing water uses that remain static over time (Bennett, pers. comm. 2015). Historically, this has been the case for the majority of groundwater-dependent projects processed by the County. This Project, however, proposes to use variable quantities of water, with intensive pumping over short periods. The intensive pumping during short periods may cause direct well interference impacts. Therefore, to evaluate potential impacts from short-term pumping of groundwater, the County Groundwater Geologist has requested a short-term drawdown analysis, in addition to the 5-year projection of drawdown, to evaluate the potential impacts from operating at the highest rate of pumping (Bennett, pers. comm. 2015).

Potential well interference impacts for Well #2 and Well #3 were evaluated over a 0.5-mile radius from each well (Figure 9, On-Site and Off-Site Wells). Table 3-4 lists known off-site active wells screened in the Jacumba Valley alluvial aquifer that are within a 0.5-mile radius of the on-site Project production wells.

| Table 3-4 Alluvial Aquifer Wells Within 0.5-Mile Radius of On-Site Project Wells |  |  |  |
| --- | --- | --- | --- |
| Well Name | Use | Distance from Well #2 | Distance from Well #3 |
|  |  | Feet |  |
| Jacumba Valley Ranch Water Company |  |  |  |
| Well Km | Public/Potable | 2,453 | 3,548 |
| Jacumba Community Services District Wells |  |  |  |
| Highland Center Well | Public/Non-Potable | 1,817 | 4,835 |
| Park Well | Public/Non-Potable | 2,256 | 5,025 |
| Other |  |  |  |
| Border Patrol Well | Private/Inactive | 1,892 | 6,235 |

Note: Bold = Well is located at a distance greater than 0.5 miles (>2,640 feet).

#### 3.2.1.2	Groundwater-Dependent Habitat

Guideline 4.2.C from the County of San Diego Guidelines for Determining Significance and Report Format and Content Requirements: Biological Resources defines the following threshold for determining a significant impact to riparian habitat or a sensitive natural community (County of San Diego 2010a):

The project would draw down the groundwater table to the detriment of groundwater-dependent habitat, typically a drop of 3 feet or more from historical low groundwater levels.

A biological field survey, including vegetation mapping, was conducted on the Project site by Dudek biologist in 2018 (Dudek 2019). The biological survey presents the most current and site-specific vegetation on the Project site and was used to identify potential groundwater-dependent habitat for the distance drawdown calculations. Vegetation and potential groundwater-dependent habitats present on the Project site are depicted in Figure 10, Potential Groundwater-Dependent Habitat. The survey identified two types of groundwater-dependent habitat, desert sink scrub and mesquite bosque.

The dominant species of the desert sink scrub are succulent chenopods, which occurs on fine-textured, poorly drained soils with high alkalinity or salt content. Characteristic species include iodine bush (Allenrolfea occidentalis), fourwing saltbush (Atriplex canescens), and salt heliotrope (Heliotropium curassavicum) (Oberbauer et al. 2008).

The dominant species of the mesquite bosque are mesquite (Prosopis glandulosa) with additional characteristic species including carelessweed (Amaranthus palmeri), white bursage, fourwing saltbush, and allscale (Oberbauer et al. 2008). Mesquite bosque commonly occur on higher alluvial terraces and near washes, streambanks, alkali sinks, or outwash plains with substantial groundwater (Dudek 2019).

The Natural Communities Commonly Associated with Groundwater (DWR 2018) and SanGIS (SanGIS 2018) vegetation dataset were also reviewed to verify potential groundwater-dependent habitat.

### 3.2.2	Aquifer Testing

The following subsections describe the procedures followed during aquifer testing at Well #2 and Well #3, and the analysis of aquifer test data.

#### 3.2.2.1	Aquifer Test Description

A 24-hour constant rate test was performed at Well #2 by Dudek on December 14, 2018, at an average pumping rate of 317 gpm. A 72-hour constant rate test was performed at Well #3 by Geosyntec on November 6, 2012, at an average pumping rate of 350 gpm (Geosyntec 2012). The purpose of the constant rate tests were to obtain approximate long-term production rates, estimate drawdown at off-site wells and groundwater-dependent habitat, and estimate aquifer properties.

#### 3.2.2.2	Aquifer Test Analysis

Aquifer Test Analysis Methodology

Hydraulic aquifer properties (transmissivity and storativity) were estimated using the computer program Aquifer Test Solver Pro, Version 4.50 (AQTESOLV). Projected drawdown was roughly estimated using drawdown data on a log-log plot. Distance drawdown was estimated at select distances from each pumping well using the Theis non-equilibrium well equation (Theis equation).

Aquifer Properties (Transmissivity and Storativity)

Aquifer transmissivity is the rate at which water flows through a vertical strip of the aquifer 1-foot wide and extending through the fully saturated thickness under a hydraulic gradient of 1, or 100%.

The aquifer coefficient of storage (also called storativity) is the volume of water released from storage per unit decline in hydraulic head in the aquifer per unit area of the aquifer. Due to well loses and inefficiency of the pumping well, an observation well is required to calculate the coefficient of storage.

Transmissivity and storativity were calculated in AQTESOLV by fitting the Cooper-Jacob (Cooper and Jacob 1953), Theis, and Neuman methods to drawdown and recovery data, where applicable.

Projected Drawdown

Groundwater drawdown was projected using the pumping rate for each aquifer test on a log-log plot. The late time trend of the drawdown curve was projected to 90 days, 1 year (365 days), and 5 years (1,825 days).

Distance Drawdown

Groundwater drawdown after 90 days, 1 year, and 5 years was estimated at the nearest off-site wells and groundwater-dependent habitat using the Theis equation (Driscoll 1986):

s=114.6 Q W(u)T

Where:

s = predicted drawdown (feet)

Q = pumping rate (gpm)

T = transmissivity (gallons per day per foot)

t = time (days)

W(u) = the well function of u

For the W(u) function, u is equal to:

u= 1.87r2STt

r = distance from pumping well (feet)

S = coefficient of storage (dimensionless)

The W(u) function, known as the Theis well function, is equal to:

Wu= -0.5772-lnu+u-u22 ∙2!+ u33 ∙3!-u44 ∙4!+…

The groundwater extraction rate used to predict drawdown was adjusted to equal the Project demand for 90 days, 1 year, and 5 years.

#### 3.2.2.3	Aquifer Test Results

Well #2 Aquifer Test

Aquifer Properties

After 24 hours of continuous groundwater extraction, the observed groundwater level drawdown was 3.1 feet in Well #2 (pumping well) and approximately 0.5 feet in Well #1 (observation well, located 305 feet away). Drawdown in Wells #2 and #1 are shown in Figures 11 and 12, respectively.

The transmissivity values obtained from the Theis and Neuman equations using AQTESOLV were 36,290 square feet per day (ft2/day) and 26,410 ft2/day in Well #1, and 33,050 ft2/day and 28,310 ft2/day in Well #2. These values were obtained using an aquifer saturated thickness (b) equivalent to 40 feet (the saturated thickness of the screened interval of Well #2). The hydraulic conductivity values calculated by dividing transmissivity by aquifer thickness (K=T/b) ranged from 660 feet per day to 907 feet per day. The storativity values estimated using data collected in Well #1 ranged from 0.028 using Theis and 0.00826 using Neuman. Table 3-5 shows the range of aquifer parameters and residual statistics obtained from the AQTESOLV curve matching of drawdown and recovery data from Wells #1 and #2. AQTESOLV results from the Well #2 aquifer test are presented in Appendix B, Well #2 Aquifer Test AQTESOLV Data.

| Table 3-5 Well #2 Aquifer Test – AQTESOLV Estimated Aquifer Hydraulic Properties |  |  |  |  |
| --- | --- | --- | --- | --- |
| Solution Method | Estimated Aquifer Hydraulic Properties Estimates |  |  | Residual Statistics |
|  | Transmissivity (square feet per day) | Hydraulic Conductivity (feet per day) | Storativity (dimensionless) | Sum of Squares (square feet) |
| Well #1 (Observation) |  |  |  |  |
| Theis | 36,290 | 907 | 0.02876 | 3.952 |
| Neuman | 26,410 | 660 | 0.00826 | 0.3775 |
| Well #2 (Pumping) |  |  |  |  |
| Theis | 33,050 | 826 | — | 11.52 |
| Neuman | 28,310 | 708 | — | 14.1 |

Note: Storativity calculated from the observation well (Well #1).

Aquifer properties estimated by using the Neuman method provided the best fit to the observation well data (see Appendix B). The transmissivity and storativity values estimated by fitting the Neuman method to Well #1 (observation well) drawdown data in AQTESOLV are 26,410 ft2/day and 0.00826, respectively. These aquifer hydraulic properties were used in the Well #2 (pumping well) distance drawdown calculation using the Theis equation.

Projected Drawdown

Projected drawdown was estimated in Wells #2 and #1 after 90 days, 1 year, and 5 years. At a constant pumping rate of 317 gpm, projected drawdown in Well #2 after 90 days, 1 year, and 5 years is 3.6 feet, 3.8 feet, and 4.0 feet, respectively (see Figure 13). Projected drawdown in Well #1 (located 305 feet away from the pumping well) at 90 days, 1 year, and 5 years is 1.49 feet, 1.81 feet, and 2.17 feet, respectively (see Figure 14).

Distance Drawdown

Distance drawdown calculations were performed at select distances from Well #2 to evaluate impacts to off-site well users and groundwater-dependent habitat after 90 days, 1 year, and 5 years of continuous groundwater extraction. The Project construction groundwater demand was analyzed over 90 days, 1 year, and 5 years. The adjusted extraction rates for distance drawdown after 90 days, 1 year, and 5 years were 282 gpm, 69 gpm, and 14 gpm (rounded), respectively. Transmissivity and storativity values used were from the Well #1 (observation well) AQTESOLV analysis (26,410 ft2/day and 0.00826, respectively).

The closest off-site well to Well #2 is the Highland Center Well located approximately 1,817 feet to the west (Figure 9, On-Site and Off-Site Wells). Projected drawdown at the Highland Center Well after 90 days, 1 year, and 5 years is predicted to be 0.86 feet, 0.27 feet, and 0.07 feet, respectively. The closest groundwater-dependent habitat to Well #2 is mesquite basque located approximately 1,820 feet south near the international border with Mexico (Figure 10). Projected drawdown at the nearest groundwater-dependent habitat after 90 days, 1 year, and 5 years is predicted to be 0.86 feet, 0.27 feet, and 0.0.07 feet, respectively. Table 3-6 summarizes projected drawdown at select distances from Well #2.

| Table 3-6 Well #2 Distance Drawdown Calculations |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nearest Off-Site Well or Groundwater-Dependent Habitat | Distance from Pumping Well #2 (feet) | Drawdown After 90 Days in Feet at a Constant Pumping Rate of 281.60 gpm | u | Drawdown After 1 Year in Feet at a Constant Pumping Rate of 69.44 gpm | u | Drawdown After 5 Years in Feet at a Constant Pumping Rate of 13.89 gpm | u |
| Highland Center Well | 1,817 | 0.86 | 0.0029 | 0.27 | 0.0007 | 0.07 | 0.0001 |
| Mesquite Bosque | 1,820 | 0.86 | 0.0029 | 0.27 | 0.0007 | 0.07 | 0.0001 |
| Park Well | 2,256 | 0.79 | 0.0044 | 0.25 | 0.0011 | 0.06 | 0.0002 |
| Well KM | 2,453 | 0.76 | 0.0052 | 0.24 | 0.0013 | 0.06 | 0.0003 |

gpm = gallons per minute; u = a ratio of distance and storavitivty over transmissivity and time. See section 3.2.2 for equation.

Wells #1 and #2 recovery data were evaluated using the plot of residual drawdown versus time since pumping started divided by time since pumping stopped (t/t’) to assess impacts to storage from pumping. At t/t’ equals to 1 (infinite time), a residual drawdown would indicate permanent dewatering or incomplete dewatering due to limited extent of the aquifer. The projected residual drawdown at infinite time for Wells #1 and #2 is 0.02 and 0.01 feet, respectively (Figures 15 and 16). This negligible residual drawdown indicates no permanent dewatering or limited extent of aquifer.

Well #3 Aquifer Test

Aquifer Properties

Aquifer properties from the Well #3 aquifer test were calculated by Geosyntec (2012). After 72 hours of continuous groundwater extraction, groundwater level drawdown was 7.3 feet in Well #3 (pumping well) and approximately 4.07 feet in the Daley Well (observation well, located 60 feet away). Drawdown in Well #3 and the Daley Well are shown in Figures 17 and 18. Aquifer properties were estimated using AQTESOLV with drawdown and recovery data recorded in Well #3 and the Daley Well (see Appendix C, Well #3 Aquifer Test Report). The transmissivity value estimated by fitting the Cooper-Jacob method (Cooper and Jacob 1953) to drawdown data recorded in the Daley Well was 8,779 ft2/day (65,821 gallons per day per foot). The transmissivity value estimated by fitting the Theis method to recovery data recorded in Well #3 was 12,950 ft2/day (96,872 gallons per day per foot). These values were obtained using an aquifer saturated thickness equivalent to 58 feet (the saturated thickness of the screened interval of Well #3), and equate to hydraulic conductivity values ranging from 151 feet per day to 223 feet per day. The storativity value estimated using data collected in the Daley Well was 0.2349 (Geosyntec 2012).

Projected Drawdown

Projected drawdown was estimated in Well #3 and the Daley Well after 90 days, 1 year, and 5 years of constantly pumping Well #3 at 350 gpm. The projected drawdown in Well #3 after 90 days, 1 year, and 5 years is 11.1 feet, 12.7 feet, and 14.5 feet, respectively (Figure 19). Projected drawdown in the Daley Well after 90 days, 1 year, and 5 years of pumping is 8.0 feet, 9.5 feet, and 11.4 feet, respectively (Figure 19) (Appendix C).

Distance Drawdown

Distance drawdown calculations were performed at select distances from Well #3 to evaluate impacts to off-site well users and groundwater-dependent habitat after 90 days, 1 year, and 5 years of continuous groundwater extraction. The Project construction groundwater demand was analyzed over 90 days, 1 year, and 5 years. The adjusted extraction rates for distance drawdown after 90 days, 1 year, and 5 years were 282 gpm, 69 gpm, and 14 gpm (rounded), respectively. The transmissivity and storativity values used were 8,779 ft2/day and 0.2349, respectively (Appendix C).

The closest off-site well to Well #3 is Well KM, owned by the Jacumba Valley Ranch Water Company, located greater than 0.5 miles (3,548 feet) to the southwest (Figure 9). Projected drawdown at Well KM after 90 days, 1 year, and 5 years is predicted to be 0.12 feet, 0.03 feet, and 0.01 feet, respectively.

The closest groundwater-dependent habitat to Well #3 is mesquite basque located 140 feet to the west (Figure 10). Projected drawdown at the nearest groundwater-dependent habitat as a result of pumping Well #3 after 90 days, 1 year, and 5 years is predicted to be 2.93 feet, 0.89 feet, and 0.22 feet, respectively. Table 3-7 summarizes projected drawdown at select distances from Well #3.

| Table 3-7 Well #3 Distance Drawdown Calculations |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Nearest Off-Site Well or Groundwater-Dependent Habitat | Distance from Pumping Well #2 (feet) | Drawdown After 90 Days in Feet at a Constant Pumping Rate of 281.60 gpm | u | Drawdown After 1 Year in Feet at a Constant Pumping Rate of 69.44 gpm | u | Drawdown After 5 Years in Feet at a Constant Pumping Rate of 13.89 gpm | u |
| Mesquite Basque | 140 | 2.93 | 0.0015 | 0.89 | 0.0004 | 0.22 | 0.0001 |
| Off-Site Groundwater Production Wells Greater than 0.5 Miles from Well #3 |  |  |  |  |  |  |  |
| Well KM | 3,548 | 0.12 | 0.9356 | 0.03 | 0.9356 | 0.01 | 0.9356 |
| Highland Center Well | 4,835 | 0.03 | 1.7374 | 0.01 | 1.7374 | 0.00 | 1.7374 |
| Park Well | 5,025 | 0.03 | 1.8766 | 0.01 | 1.8766 | 0.00 | 1.8766 |

gpm = gallons per minute; u = a ratio of distance and storavitivty over transmissivity and time. See section 3.2.2 for equation.

Daley Well and Well #3 recovery data were evaluated using the plot of residual drawdown versus time since pumping started divided by time since pumping stopped (t/t’) to assess impacts to storage from pumping. The projected residual drawdown at infinite time for the Daley Well and Well #3 is 0.5 feet (Figures 20 and 21). This residual drawdown is less than the County’s standard (County of San Diego 2007) of more than 0.5 feet of residual drawdown that would indicate permeant dewatering or limited aquifer extent.

### 3.2.3	Significance of Impacts Prior to Mitigation

A pumping rate of 317 gpm from the Well #2 aquifer test and of 350 gpm from the Well #3 aquifer test were used to predict Project drawdown using each well’s maximum pumping rate. These pumping rates equate to maximum annual production of approximately 511 afy from Well #2 and 564 afy from Well #3. The maximum annual production rates calculated for Well #2 and Well #3 are significantly greater than the Project water demand of 112 acre-feet of water during Project construction (1 year), 10 afy for ongoing O&M (approximately 38 years), and 50 acre-feet for decommissioning and dismantling (1 year).

To assess the potential for Project groundwater extraction to draw down the groundwater table to the detriment of nearby groundwater-dependent habitat, or to cause well interference, projected drawdown within a 0.5-mile radius of Wells #2 and #3 was estimated using the Theis equation. Periods of 90 days, 1 year, and 5 years were used to calculate the potential long-term impacts to nearby groundwater-dependent habitats and domestic and public pumping wells. Pumping rates for each well were adjusted to reach total Project construction demand at the end of 90 days, 1 year, and 5 years.

Based on the drawdown calculations performed, drawdown at the closest off-site groundwater well to Well #2, the Highland Center Well, after 90 days, 1 year, and 5 years of pumping is predicted to be 0.86 feet, 0.27 feet, and 0.07 feet, respectively. Drawdown at the nearest groundwater-dependent habitat to Well #2 (located approximately 1,820 feet south) after 90 days, 1 year, and 5 years of pumping is predicted to be 0.86 feet, 0.27 feet, and 0.07 feet, respectively.

No groundwater wells are located within a 0.5-mile radius of Well #3. The nearest off-site production well is Well Km, located 3,548 feet (greater than 0.5 miles) southwest of Well #3. The projected drawdown at Well Km from Well #3 pumping after 90 days, 1 year, and 5 years is predicted to be 0.12 feet, 0.03 feet, and 0.01 feet, respectively. Drawdown at the nearest groundwater-dependent habitat to Well #3 (located approximately 140 feet west) after 90 days, 1 year, and 5 years of pumping is predicted to be 2.93 feet, 0.89 feet, and 0.22 feet, respectively.

Current groundwater levels near Well #2 and Well #3 are at least 12 feet higher than the historical low groundwater level recorded in the Jacumba Valley alluvial aquifer (Exhibit 2-2, Well K3). Well #2 and #3 pumping is not expected to draw down the groundwater table greater than 3 feet from the historic low.

Based on the Theis methods, the effects of Project pumping on nearby groundwater-dependent vegetation and off-site domestic and public pumping wells is anticipated to be less-than-significant. Project pumping is not anticipated to adversely impact nearby groundwater-dependent vegetation or cause well interference. Additionally, the analysis performed is a conservative approach, since it likely overestimated predicted drawdown. This is because the calculations assumed no rainfall recharge to occur over the time periods tested. Recharge will offset groundwater-level decline related to groundwater extraction during periods of above-average annual rainfall (non-drought conditions).

### 3.2.4	Mitigation Measures and Design Considerations

A GMMP will be prepared to monitor groundwater thresholds for off-site well interference and groundwater-dependent habitat.

### 3.2.5	Conclusions

The analysis above indicates that the potential for Project groundwater extraction from Wells #2 and #3 to impact off-site wells or nearby groundwater-dependent habitat is anticipated to be less-than-significant. For safe measure, groundwater-level monitoring would be performed in several wells to record groundwater levels during groundwater extraction. A GMMP detailing groundwater thresholds for off-site well interference and groundwater-dependent habitat will be prepared. Annual review of groundwater-level data would be conducted by a Certified Geologist or Hydrogeologist registered in the State of California to evaluate long-term impacts.

# 4	Water Quality Impact Analysis

The Project does not propose to use groundwater as a potable water source; therefore, no water quality impact analysis was conducted.

INTENTIONALLY LEFT BLANK

# 5	Summary of project Impacts and Mitigation

## 5.1	50% Reduction in Groundwater Storage

As discussed in Section 3.1, 50% Reduction of Groundwater Storage, a Project-specific soil-moisture-based water balance was not performed for the Project site. Instead, a 1-year Project construction groundwater extraction volume of 112 acre-feet was compared to historical, ongoing, and future estimated groundwater extraction rates from the Jacumba Valley alluvial aquifer and updated estimates of groundwater in storage originally made by Roff and Franzone (1994) and Swenson (1981). The analysis evaluated whether the water demands for Project construction, ongoing groundwater extraction, and maximum non-potable extraction by JCSD maintain at least 50% groundwater in storage over the 2,060-acre Jacumba Valley alluvial aquifer. The analysis evaluated groundwater extraction of 112 acre-feet for Project construction, 8 afy for ongoing domestic and Jacumba Valley Ranch Water Company use, 119.5 afy for JCSD potable supply, and a maximum pumping capacity of 410 afy from JCSD non-potable wells. The total water demand of 649.5 acre-feet for these projects is 7.2% of the current estimated groundwater storage of the Jacumba Valley Alluvial aquifer. The analysis concluded that groundwater extraction for the Project and for ongoing and future water demands would maintain at least 50% groundwater in storage.

Total groundwater extraction over the assumed lifetime of the Project was also analyzed. The total estimated Project groundwater extraction for the 40-year lifetime (542 acre-feet) of the Project is 6% of the current groundwater in storage. Since the Project would not exceed the 50% reduction in groundwater storage threshold, and other cumulative groundwater demands would be met, groundwater impacts to storage would be less than significant.

## 5.2	Well Interference

As presented in Section 3.2.2.2, Aquifer Test Analysis, based on the Theis equation, drawdown at the closest off-site groundwater well to Well #2, the Highland Center Well, after 90 days, 1 year, and 5 years of pumping is predicted to be 0.86 feet, 0.27 feet, and 0.07 feet, respectively (Table 3-6). No groundwater wells are located within a 0.5-mile radius of Well #3. These results indicate that drawdown is not predicted to exceed the County well interference threshold of significance of a decrease in water level of 5 feet or more in off-site alluvial wells (County of San Diego 2007).

## 5.3	Groundwater-Dependent Habitat

As presented in Section 3.2.1.2, Mesquite Bosque located approximately 1,820 feet south of Well #2 is potentially groundwater-dependent habitat. Based on the Theis equation, drawdown at the closest groundwater-dependent habitat to Well #2 after 90 days, 1 year, and 5 years is predicted to be 0.86 feet, 0.27 feet, and 0.07 feet, respectively (Table 3-6).

Mesquite Basque located approximately 410 feet west of Well #3 is potentially groundwater-dependent habitat. Based on the distance drawdown calculations, drawdown at the closest groundwater-dependent habitat to Well #3 after 90 days, 1 year, and 5 years of pumping is predicted to be 2.93 feet, 0.89 feet, and 0.22 feet, respectively. Current groundwater levels in Well #3 are at least 12 feet higher than the historical low groundwater level recorded in the Jacumba Valley alluvial aquifer (Exhibit 2-2, Well K3). Therefore, the Project is unlikely to draw down the groundwater table to the detriment of groundwater-dependent habitat, and impacts are anticipated to be less than significant.

## 5.4	Mitigation Measures

Project production wells, Well #2 and Well #3, should be fitted with totalizing flow meters to record production during each phase of the Project. Groundwater wells should also have access for taking groundwater-level measurements. Monitoring would be in place during groundwater production for Well #2 and Well #3 to monitor impacts to groundwater storage, well interference, and groundwater-dependent habitat. A GMMP would be prepared that details groundwater thresholds for off-site well interference and groundwater-dependent habitat.

# 6	REFERENCES

AECOM. 2011. Energia Sierra Juarez (ESJ) Well Access Road – Project Number 09-0107420 Biological Letter Report. February 3, 2011.

Allan, R.B. 2013. Unpublished precipitation data from Jacumba and Tierra del Sol rain gauges. Personal communication from R. Allan (County of San Diego Flood Control). September 13, 2013.

Allan, R.B. 2014. Unpublished precipitation data from Tierra del Sol rain gauges. Personal communication from R. Allan (County of San Diego Flood Control). September 24, 2014.

Barrett (Barrett Consulting Group). 1996. Hydrogeological Investigation, Proposed Jacumba Valley Ranch Development, Jacumba, San Diego County, California. Prepared for Jacumba Valley Ranch. October 18, 1995.

Bennett, Jim. 2015. Discussion of Guidelines for Determining Significance and Report Format and Content Requirements: Groundwater Resources. Relayed to Trey Driscoll (Dudek).

CIMIS (California Irrigation Management Information System). 1999. “ETo Zones Map.” CIMIS. 1999. Accessed August 2013. http://www.cimis.water.ca.gov/
cimis/cimiSatEtoZones.jsp.

Cooper, H.H., Jr., and C.E. Jacob. 1953. “A Generalized Graphical Method of Evaluating Formation Constraints and Summarizing Well Field History.” U.S. Department of the Interior, Geological Survey, Water Resources Division, Groundwater Branch. January 1953.

County of San Diego. 2004. Groundwater Limitations Map. On file with the Clerk of the Board of Supervisors as Document No. 195172. April 2, 2004.

County of San Diego. 2007. County of San Diego, Guidelines for Determining Significance and Report Format and Content Requirements: Groundwater Resources. Land Use and Environment Group, Department of Planning and Land Use, Department of Public Works. March 19, 2007.

County of San Diego. 2010a. County of San Diego, Guidelines for Determining Significance and Report Format and Content Requirements: Biological Resources. Land Use and Environment Group, Department of Planning and Land Use, Department of Public Works. Fourth Revision. September 15, 2010.

County of San Diego. 2010b. General Plan Update – Final Groundwater Study. Department of Planning and Land Use. April 2010.

County of San Diego. 2011. San Diego County General Plan: A Plan for Growth, Conservation, and Sustainability. Mountain Empire Subregional Plan. Adopted August 3, 2011.

County of San Diego. 2018. Department of Environmental Health. Environmental Health Document Search. Accessed 2018. https://www.sandiegocounty.gov/content/sdc/
deh/doclibrary/.

CRA (Conestoga-Rovers and Associates). 2012. Monitoring Well Installation and Soil Boring Report Former Chevron Service Station 44485 Old Highway 80, Jacumba, California. Case No. H29832-002. May 1, 2012.

Driscoll, Fletcher G. 1986. Groundwater and Wells. St Paul, Minn: Johnson Division.

Devine, B. 2019. Jacumba Community Services District Service Connections. Personal communication from B. Devine (General Manger Jacumba Community Services District).

Dudek. 2015. Groundwater Resources Investigation Report – Flat Creek Watershed. Jacumba Community Services District. April 2015.

Dudek. 2016a. Draft Highland Center Well Completion Report. Prepared for Jacumba Community Services District. November 2016.

Dudek. 2016b. Final Preliminary Engineering Report Proposed Domestic Water Supply Project – Manganese Treatment System Water Supply Wells No. 7 and 8, Photovoltaic Array and Pipeline Replacement Prepared for the Jacumba Community Services District and U.S. Department of Agriculture – Rural Development. August 2016.

Dudek. 2019. Draft Biological Resources Technical Report for the JVR Energy Park

DWR (California Department of Water Resources). 2004. Jacumba Valley Groundwater Basin. California’s Groundwater. Bulletin 118. February 2004.

DWR. 2018. Natural Communities Commonly Associated with Groundwater dataset. Accessed November 2018. https://data.ca.gov/dataset/natural-communities-commonly-associated-groundwater.

DWR and UCCE (University of California Cooperative Extension). 2000. A Guide to Estimating Irrigation Water Needs of Landscape Plantings in California, The Landscape Coefficient Method and WUCOLS III. Accessed August 2013. http://www.water.ca.gov/
wateruseefficiency/docs/ wucols00.pdf.

EnviroMatrix Analytical. 2015. “Jacumba Park Monitoring Well.” Results of Analysis of Samples. November 18, 2015.

Garrity, C.P., and D.R. Soller. 2009. Database of the Geologic Map of North America; adapted from the map by J.C. Reed, Jr. and others (2005): U.S. Geological Survey Data Series 424. https://pubs.usgs.gov/ds/424/.

Geosyntec. 2012. Jacumba Valley Ranch Property Well #3 Aquifer Test Report, Jacumba, CA. November 2012.

Google Earth. 2015. Aerial Photography Imagery. August 28, 2015.

Holland, R.F. 1986. Preliminary Descriptions of the Terrestrial Natural Communities of California. State of California, The Resources Agency, Nongame Heritage Program, Dept. Fish & Game, Sacramento, Calif. 156 pp.

Integrated Urban Forestry. 2001. Adaptability of Native Plant Species to Groundwater Fluctuations for Sycamore Ranch. Laguna Hills, California. 2001.

LAFCO (Local Agency Formation Commission). 2013. Independent and Dependent Districts. Jacumba Community Services District. Updated September 16, 2013. http://www.sdlafco.org/images/Profiles/Profile_CSD_Jacumba.pdf.

Leighton (Leighton and Associates Inc.). 1991a. Limited Evaluation of Liquefaction and Consolidation Potential, Phase 1, Jacumba Valley Ranch Development, San Diego County, California. January 21, 1991.

Leighton. 1991b. Updated Evaluation of Consolidation Potential, Phase 1, Jacumba Valley Ranch Development, San Diego County, California. February 27, 1991.

McCullough, Jamelle. 2015. Jacumba ValleyRanch Water Company Connection Information. Personal communication from Jamelle McCullough (County of San Diego Department of Environmental Health, Small Drinking Water Systems). April 8, 2015.

National Research Council. 2002. Riparian Areas – Functions and Strategies for Management, Committee on Riparian Zone Functioning and Strategies for Management. Water Science and Technology Board, Board on Environmental Studies and Toxicology, Division on Earth and Life Studies. National Research Council. National Academy Press, 2002.

NOAA (National Oceanic and Atmospheric Administration). 2011. National Climatic Data Center (NCDC) Boulevard Surface Data Daily 12/1924 to 12/1967, Surface Data Monthly 1931 to 1967. Boulevard 2 Surface Data Daily 10/1969 to 12/1994, Surface Data Monthly 1969 to 1994. Accessed October 2011. http://www.ncdc.noaa.gov.

NRCS (National Resource Conservation Service). 2018. Plant Database. Accessed December 2018. http://plants.usda.gov/java/.

Oberbauer, T., M. Kelly, and J. Buegge. 2008. Draft Vegetation Communities of San Diego County. Based on “Preliminary Descriptions of the Terrestrial Natural Communities of California,” by R.F. Holland, PhD, October 1986. March 2008.

Pape, Bill. 2015. Informal discussion of the historical water demand of Bornt Farms and Jacumba Valley Ranch Water Company. Relayed to T. Driscoll (Dudek).

Peterson, J. 2014. Jacumba Valley Ranch Groundwater Level Data. Personal communication from J. Peterson (Peterson Environmental Services). 2014.

Petra (Petra Environmental Division). 2006. Well Installation, Testing and Sampling Report, Jacumba Community Services District. 44465 Old Highway 80, Jacumba, California. June 30, 2006.

Robinson, T.W. 1952. “Phreatophytes and their Relation to Water in Western United States.” Transactions American.

Robinson, T.W. 1958. “Phreatophytes: Geological Survey Water Supply Paper 1423.” United States Department of the Interior.

Roff, D.F., and J.G. Franzone. 1994. Proposed Final Report, Hydrogeological Investigation, Proposed Jacumba Valley Ranch Development, Jacumba, San Diego County, California. Leighton and Associates Inc. 19 p.

SanGIS (San Diego Geographic Information Source). 2018. Vegetation Information in the San Diego Region. Shapefile. Accessed 2018.

SDCWA (San Diego County Water Authority). 2010. San Diego County Water Authority Subregional Natural Community Conservation Plan/Habitat Conservation Plan. Section 4, Plan Area Biology and Land Use. https://www.sdcwa.org/sites/default/files/
files/NCCP-HCP-merged.pdf.

Swenson, G.A. 1981. Master’s Thesis San Diego State University – The Groundwater Hydrology of Jacumba Valley, California and Baja California.

Troutt, D. 2015. JCSD Groundwater Production and Water Level Data. Personal communication from D. Troutt (Former General Manger Jacumba Community Services District). 
January 12, 2015.

WRCC (Western Regional Climate Center). 2019. “Period of Record General Climate Summary—Temperature: Campo, California.” WRCC. Accessed September 2019. http://www.wrcc.dri.edu/cgi-bin/cliMAIN.pl?ca1424.

U.C. Davis (University of California at Davis, Vegetable Research and Information Center). 2011. “Iceberg Lettuce Production in California.” Publication 7215. 2011.

USDA (U.S. Department of Agriculture). 2015. National Resource Conservation Service, Custom Soil Resource Report for San Diego County Area, California, based on San Diego County Survey Area, Version 6, December 17, 2007. Accessed March 2015. http://websoilsurvey.nrcs.usda.gov.

USGS (U.S. Geological Survey). 2004. “Preliminary Geologic Map of the El Cajon 30' x 60' Quadrangle, Southern California.” Version 1.0. 1:100,000. Compiled by V.R. Todd.

INTENTIONALLY LEFT BLANK

# 7	List of preparers and persons anD 
organizations contacted

This report was prepared by Dudek Hydrogeologist Trey Driscoll, PG, CHG, who is a County of San Diego–approved hydrogeologist. Dudek hydrogeologist Hugh McManus conducted fieldwork, report preparation, graphics, and GIS mapping. Dudek hydrogeologist Devin Pritchard-Peterson performed aquifer test data analysis and preparation of associated graphics, and composed sections of this report. Peer review was provided by Kayvan Ilkhanipour, PG, CHG. This report was prepared in coordination with County Groundwater Geologist Jim Bennett with meteorological input from Rand Allan from the San Diego County Flood Control. Billy Devine, General Manger, Jacumba Community Services District, assisted with background information and data for this report.

INTENTIONALLY LEFT BLANK

Figure 1	Regional Location

INTENTIONALLY LEFT BLANK

Figure 2	Vicinity Map

INTENTIONALLY LEFT BLANK

Figure 3	Hydrologic Areas

INTENTIONALLY LEFT BLANK

Figure 4	Regional Mean Annual Precipitation

INTENTIONALLY LEFT BLANK

Figure 5	Current General Plan Land Use

INTENTIONALLY LEFT BLANK

Figure 6	Regional Geologic Map

INTENTIONALLY LEFT BLANK

Figure 7	Soils Map

INTENTIONALLY LEFT BLANK

Figure 8	Hydrogeologic Units

INTENTIONALLY LEFT BLANK

Figure 9	On-Site and Off-Site Wells

INTENTIONALLY LEFT BLANK

Figure 10	Potential Groundwater-Dependent Habitat

IINTENTIONALLY LEFT BLANK

Figure 11	Well #2 24-Hour Constant Rate Test: Well #2 Drawdown

INTENTIONALLY LEFT BLANK

Figure 12	Well #2 24-Hour Constant Rate Test: Well #1 Drawdown

INTENTIONALLY LEFT BLANK

Figure 13 	Well #2 24-Hour Constant Rate Test: Well #2 Projected Drawdown

INTENTIONALLY LEFT BLANK

Figure 14	Well #2 24-Hour Constant Rate Test: Well #1 Projected Drawdown

INTENTIONALLY LEFT BLANK

Figure 15	Well #2 24-Hour Constant Rate Test: Well #2 Recovery

INTENTIONALLY LEFT BLANK

Figure 16	Well #2 24-Hour Constant Rate Test: Well #1 Recovery

INTENTIONALLY LEFT BLANK

Figure 17 	Well #3 72-Hour Constant Rate Test: Well #3 Drawdown

INTENTIONALLY LEFT BLANK

Figure 18	Well #3 72-Hour Constant Rate Test: Daley Well Hydrograph

INTENTIONALLY LEFT BLANK

Figure 19 	Well #3 72-Hour Constant Rate Test: Well #3 and Daley Well Projected Drawdown

INTENTIONALLY LEFT BLANK

Figure 20 	Well #3 72-Hour Constant Rate Test: Daley Well Recovery

INTENTIONALLY LEFT BLANK

Figure 21	Well #3 72-Hour Constant Rate Test: Well #3 Recovery

INTENTIONALLY LEFT BLANK

Appendix A

Well Completion Information

Appendix B

Well #2 Aquifer Test AQTESOLV Data

Appendix C

Well #3 Aquifer Test Report

Studies have found that groundwater elevation reductions adversely affect native plant species. Two of the referenced studies (Integrated Urban Forestry 2001 and National Research Council 2002) found that a permanent reduction in groundwater elevation of greater than 3 feet is enough to induce water stress in some riparian trees, particularly willow (Salix spp.), cottonwood (Populus spp.), and Baccharis species.

Non-potable groundwater extraction from the Highland Center Well and the Park Well is based on totalizer readings collected during routine groundwater monitoring performed by Dudek staff as required for the Jacumba Solar Groundwater Monitoring and Mitigation Plan.

The estimate of 9,005 acre-feet of groundwater in storage in 2018 for the Jacumba Valley alluvial aquifer is an initial estimate based on available data, including well logs, water levels, and aquifer properties estimated by pump testing. The estimated storage in the Jacumba Valley alluvial aquifer may be revised as additional data is acquired.

Historical groundwater level hydrographs compiled by the Jacumba Community Sponsor Group –Town Center Well Hydrographs from 1990 to 2008 indicate up to 20 feet of water level decline in one well during this period of measurement (Figure 2-58 in County of San Diego 2010b). Historical groundwater level monitoring for JCSD Well #4 from 1990 to 2008 indicates up to 20 feet of water level decline during the period of measurement.