to:	Ed Gowens

from:	Nicholas koutoufidis, STAFF airport SPECIALIST

subject: 	PDS2018-GPA-18-010 (JVR SOLAR) comMENTS – airport review

date:	February 28, 2019

CONSISTENCY DETERMINATION:

Open Land Criteria

Pursuant to Section JAC.2.8.a. of the Jacumba ALUCP, “On project sites of 10 acres or more, structures and other large objects shall be arranged as to meet the open land criteria in Policy JAC.2.9 below at the rate of one open land area per each 10 acres of the site.”

Staff analyzed the project (see Attachment A) and found that the project impacts approximately 122 acres of land within Safety Zones 2 and 4 of the Jacumba ALUCP.

Pursuant to the Jacumba ALUCP, this would require 12 Open Land Areas (6.1 acres). The project provides approximately 8.8 acres of Open Land that meet the criteria of having minimum dimensions of 75 feet by 300 feet and is oriented with the typical direction of aircraft. Pursuant to Section JAC.2.9.d., roads are acceptable as open land areas if they meet the criteria.

Lot Coverage

In reference to lot coverage, there is no definition of lot coverage in the ALUCP or in the California Airport Land Use Planning Handbook.

The County of San Diego Zoning Ordinance defines lot coverage as “the percentage of net site area covered by the vertical projection of any structure excluding any structure not extending above grade.”

Under this definition, the area between each solar panel cannot be calculated as lot coverage because there is no vertical projection above grade. Therefore, the 25-45% calculation that the applicant provided is sufficient to meet the maximum lot coverage policy.

The project is consistent with the Jacumba Airport Land Use Compatibility Plan and should be approved by the Airport Land Use Commission.